Livewire System Control Protocol

Revision History

Jan 07 – Mar 09, 2005	V.01E1	Gints Linis: created

Mar 17 – Mar 23, 2005	V.02E1	Gints Linis:

Consolidated changes, proposed during discussion:

- Reworked event philosophy, introduced per-event properties, eliminated event ID as the default property: edited 3.4, 3.5, 5.2.4, 5.2.5, 5.2.6, 5.5.5, 5.5.6, 5.5.7, 6.4, 6.5, 6.7; eliminated 5.3.1

- Entirely eliminated the default property mechanism: edited 4.10.3

- $IND option in SET commands: edited 5.2.1, 5.2.3, 5.5.2; new 5.4.6

- Refinements to the logic associated with ACK messages: edited 5.2.7, 5.5.2

- Floating point numbers are made optional, to a convention: edited 4.6

- Multidimensional arrays redefined as a special case of nested lists: edited 4.5

Declined proposals:

- Eliminate EVENT messages, use INDI instead

- Use SUB to request INDI responses for SET commands

- Use white space for list separator instead of comma

- Use “–” as the system name discriminator instead of “$”

- Move system items to second position, after the operation name

Other changes:

- Formal statement of GPIO protocol application scope: inserted 6.1

- GPIO outputs do not generate notifications: edited 6.6

- Code ranges and new values defined: edited Appendix A

User Panel Application Protocol

7 – New sub-protocol chapter written

Jul 14, 2005	V.03E1	Gints Linis:

New LOGIN operation:

5.1, 5.2.1 – LOGIN command listed and described

5.3.1, 5.3.2, 5.3.3 – New USER, PWD, ACCLEV properties

5.5.2 – New message sequence examples

Color control refined:

7.3.6 – property names changed, 4 independent fore-back color schemes

Console Remote Control Protocol

8 – New sub-protocol chapter written

Resource Discovery Protocol

9 – New sub-protocol chapter written

Jul 19, 2005	V.04E1	Gints Linis:

Common framework definition extended:

4.5 – Literal constants may include line breaks

User panel protocol: Ch.7 – multiple refinements

- Names changed: MOD_USER, BUT, KEY, IND

- New DISP control type

- IND property value type changed

- Using line breaks in TEXT property illustrated

- Color control philosophy explained

- Color control properties redefined

- Hex presentation allowed for RGB colors

Apr 07, 2006	V.05E1	Gints Linis:

Remote control - accessory module messages

8.2.2 – New object class: source channel

8.3.5 to 8.3.8 – Properties for the source channel

8.4.6, 8.4.7 – Source channel operations

8.5 – approach for using UDP multicast explained

8.6.2 – Message sequences for source channel operations

Jul 17, 2006	V.06E1	Gints Linis:

Document general:

8.2 – Sections in this chapter grouped by remote control objects (8.)

Protocol framework modification:

5.2.6 – default for SUB command is all properties, not only events (5.2.6)

New features:

RQ 3325: Remote control – V-Mixer

8.4 – New V-Mixer section (8.4)

Jun 15, 2007	V.07E1	Gints Linis:

Documentation errors corrected:

8.4.3.5 – Typo corrected: INDI  EVENT (8.4.3.5)

New features:

RQ 3604: VMIX – engine local control

8.4 – Section edited to reflect the changed control approach (8.4)

RQ 3757: Remote control – V-Mode

8.5 – New V-Mode section (8.5)

Jun 28, 2007	V.08E1	Gints Linis:

Protocol framework changes:

4.3 – Inserting the “space” symbols is allowed before and after the list separator “comma” (4.3)

Document updated to match the actual implementation:

8.4 – Class name “CH” renamed to “LWCH” (8.4)

Document updated to cover implemented features:

RQ 3708/T-56: Reporting fader channel ON and OFF button events

8.3 – Included in the new section: Fader channel – basic operations (8.3)

RQ 3431/T-51: Setting fader channel ON, OFF,  MUTE; reporting state events

8.3 – Included in the new section: Fader channel – basic operations (8.3)

New features:

RQ 3759/T-88: Setting fader gain

8.3 – Included in the new section: Fader channel – basic operations (8.3)

RQ 3776/T-97: Fader assignments to PGMx

8.3 – Included in the new section: Fader channel – basic operations (8.3)

Jul 04, 2008	V.09	Gints Linis:

Documentation corrected:		v.09

5.2.1 – Added missing quotation marks

8.4.1 – Added missing port allocation info: port 4011

New features:

RQ 3994: Supervision status monitoring		RQ 3994

10 – New sub-protocol chapter

Jul 09, 2008	V.10	Gints Linis:

Refinements in existing mechanisms:

RQ 4176: Password protection – general logic   RQ 4176

3.7 – New section: access control logic

5.2.1 – LOGIN message description refined

5.3.1 – New UNIT object introduced

5.6.2 – Message sequence examples for LOGIN procedures extended

Jul 22, 2008	V.10a	Gints Linis:

Documentation corrected:		v.10a

5.6.2 – Added missing quotation marks

Sep 04, 2008	V.10b	Gints Linis:

RQ 3419: Omnia processing on PGM1/Mon1 Headphones   RQ 3419

8.6.2.2 – New selector state value for “PGM1 MON Processed”

Feb 24, 2009	V.11	Gints Linis:

RQ 4519: VMIX and VMODE input connections   RQ 4519

8.5.2, 8.5.3, 8.6.2, 8.6.3 – New “input_uri” property, command sequences

Sep 21, 2009	V.12	Gints Linis:

Documentation general:		v.12

4.2 – ambiguous statements clarified

8.3 – Scope of the section extended from only faders to all console modules

RQ 3692: Fader source selection   				RQ 3692

8.3.3, 8.3.4.6 – New console input source related properties and handling

9.3.2, 9.4.2 – New global source list property and handling

Appendix B – New source status encoding reference

RQ 3777: Monitor external and aux return source selection   RQ 3777

8.3.2.3, 8.3.2.4, 8.3.2.5, 8.3.2.6 – New objects for aux returns and monitors

8.3.3, 8.3.4.7 – New console input source related properties and handling

RQ 4610: Querying console resources   			RQ 4610

8.2.2, 8.2.3.6 – New fader list property and handling

9.3.4, 9.4.4 – New console hardware description property and handling

RQ 4736: Monitor module button events   			RQ 4736

8.3.3, 8.3.4.9 – New monitor source selector button state properties and handling

RQ 4738: Aux returns - on/off, fader gain, PGMx assignments   RQ 4738

8.3.3, 8.3.4 – Applicability of the relevant properties extended to aux returns

Contents

1.	Introduction	3

1.1	Scope	3

1.2	Definitions and abbreviations	8

1.3	References	8

2.	Design goals and principles	8

2.1	Supporting concurrent carrier protocols	8

2.2	Future extensions and coexistence of protocol variants	8

2.3	Connections and transport services	9

2.4	Universal message composition	9

2.5	Unambiguous message interpretation	9

2.6	Universal operation types	10

2.7	Application area specific items	10

3.	General design	10

3.1	Application entities	10

3.2	Transport connections	10

3.3	Objects and properties	11

3.4	Events	11

3.5	Subscription to data flows	12

3.6	Livewire channels	12

3.7	Access control	13

3.7.1	General logic	13

3.7.2	09 Jul 2008:   The current requirements	13

4.	Message composition	14

4.1	Character set	14

4.2	Case sensitivity	14

4.3	Special symbols	14

4.4	Names	15

4.5	Property values	15

4.6	Numeric data presentation	16

4.7	Encapsulated data	16

4.8	Message delimitation	16

4.9	Message structure	17

4.10	Construction of message elements	17

4.10.1	Operation	17

4.10.2	Object	17

4.10.3	Property list	18

4.10.4	System items	18

5.	Universal operations	18

5.1	Universal operations summary	18

5.2	Message descriptions	19

5.2.1	LOGIN: Login request	19

5.2.2	SET: Set value	19

5.2.3	GET: Read value	20

5.2.4	INDI: Indicate value	20

5.2.5	EVENT: Event notification	20

5.2.6	SUB: Subscribe to data flow	21

5.2.7	UNSUB: Unsubscribe from data flow	21

5.2.8	ACK: Acknowledgement	22

5.3	Universal objects	22

5.3.1	UNIT: The root object	22

5.4	Universal properties	23

5.4.1	USER: User name for login	23

5.4.2	PWD: Password for login	23

5.4.3	ACCLEV: Access level for login	23

5.5	Universal system items	23

5.5.1	$PROTID: Protocol version identifier	23

5.5.2	$MSGI: Message identifier	24

5.5.3	$TRXI: Transaction identifier	24

5.5.4	$STATUS: Status code	25

5.5.5	$ACK: Force acknowledgement	25

5.5.6	$IND: Force indication	25

5.6	Message sequence examples	26

5.6.1	Notation	26

5.6.2	Login	26

5.6.2.1	Unprotected interface	26

5.6.2.2	Protected interface, successful login, closing the connection resets the rights	26

5.6.2.3	Protected interface, repeated login procedures during a single connection	26

5.6.2.4	Unsupported access level parameter is ignored	27

5.6.2.5	Local client always has the full rights	27

5.6.2.6	Status responses	27

5.6.3	Set property value	27

5.6.4	Read property value	28

5.6.5	Indicate property value	28

5.6.6	Broadcast event notification	28

5.6.7	Subscribe to data flow	28

5.6.8	Unsubscribe from data flow	29

6.	GPIO Application Protocol	31

6.1	Application	31

6.2	GPIO application objects	31

6.3	Properties	31

6.3.1	GPI STATE: State of input circuit	31

6.3.2	GPO STATE: State of output circuit	32

6.3.3	GPO PULSE: Command to output circuit	32

6.4	Conventions and options	32

6.5	GPIO events	33

6.6	GPIO commands	34

6.7	GPIO configurations	35

6.7.1	Console – source/monitor, UDP multicast	35

6.7.2	Console – source/monitor, TCP unicast	36

6.7.3	GPIO snake, TCP unicast	36

6.8	Proposed approaches for overcoming hardware capacity limitations	37

7.	User Panel Application Protocol	39

7.1	Application	39

7.2	User panel application objects	39

7.2.1	General	39

7.2.2	Object addressing models	39

7.2.3	Control types	40

7.2.3.1	Button	40

7.2.3.2	Text/bitmap display	40

7.2.3.3	Other control types - TBD	41

7.3	Properties	41

7.3.1	KEY: State of the key	41

7.3.2	IND: State of the indication device	41

7.3.3	TEXT: ASCII characters to display	42

7.3.4	BITMAP: Bitmap pattern to display	43

7.3.5	ICON: Icon to display	44

7.3.6	FORECOLORON: foreground color for ON state BACKCOLORON: background color for ON state FORECOLOROFF: foreground color for OFF state BACKCOLOROFF: background color for OFF state	44

7.4	Message sequence examples	45

7.4.1	Key events	45

7.4.2	Commands to indication device	46

8.	Console Remote Control Protocol	48

8.1	Application	48

8.2	Common application control	48

8.2.1	Objects	48

8.2.2	Properties	48

8.2.2.1	Overview	48

8.2.2.2	ShowProfID: Show profile identifier	48

8.2.2.3	ShowProfName: Show profile name	49

8.2.2.4	ShowProfStat: Show profile loading status	49

8.2.2.5	GpioList: GPIO resource list	49

8.2.2.6	fader_list: List of the actually installed fader numbers	50

8.2.3	Commands and events	50

8.2.3.1	Legend	50

8.2.3.2	Trigger a profile change	50

8.2.3.3	Determine what the current profile and its status is	50

8.2.3.4	Observe profile changes	50

8.2.3.5	Obtain a list of GPIO resources	50

8.2.3.6	Obtain a list of actually installed physical fader numbers	51

8.2.4	Message sequence examples	51

8.3	Console operations	52

8.3.1	Application	52

8.3.2	Objects	52

8.3.2.1	Source ID – Livewire channel	52

8.3.2.2	Physical fader channel	52

8.3.2.3	Aux returns	53

8.3.2.4	Monitor 1	53

8.3.2.5	Monitor 2	53

8.3.2.6	Preview	53

8.3.3	Properties	54

8.3.3.1	Overview	54

8.3.3.2	ON/OFF state	54

8.3.3.3	Fader channel input muting state	55

8.3.3.4	Fader channel mix assignment states	55

8.3.3.5	Fader gain	55

8.3.3.6	Command button state properties	55

8.3.3.7	Source profile list	56

8.3.3.8	Source profile identifier	56

8.3.3.9	Source profile name	56

8.3.3.10	Source profile loading status	57

8.3.3.11	Source Livewire channel	57

8.3.4	Commands and events	57

8.3.4.1	Legend	57

8.3.4.2	Subscribe to fader or aux return input channel control state changes	58

8.3.4.3	Set fader channel controls, addressing by source LW channel number	58

8.3.4.4	Read fader channel controls, addressing by source LW channel number	59

8.3.4.5	Notify remote application about fader channel control state changes, addressing by source LW channel number	59

8.3.4.6	Set, get, observe fader and aux return input channel controls, addressing by physical input channel	59

8.3.4.7	Fader source handling	60

8.3.4.8	Aux return and monitor input source handling	60

8.3.4.9	Notify remote application about monitor source selector button state changes	61

8.4	Fader channel – accessory module operations	61

8.4.1	Application	61

8.4.2	Objects	61

8.4.2.1	Source ID – Livewire channel	61

8.4.3	Properties	61

8.4.3.1	Overview	61

8.4.3.2	Command button state properties	62

8.4.3.3	Command button lamp properties	62

8.4.3.4	Headphones selector rotation amount	63

8.4.3.5	Headphones source name display text	63

8.4.4	Commands and events	63

8.4.4.1	Legend	63

8.4.4.2	User command from accessory module	63

8.4.4.3	Indication to accessory module	64

8.4.5	Message sequence examples	64

8.5	V-Mixer	65

8.5.1	Objects	65

8.5.2	Properties	65

8.5.2.1	Overview	65

8.5.2.2	Submix group master gain	66

8.5.2.3	Input source	66

8.5.2.4	Input gain	66

8.5.2.5	Input ramp switch state	67

8.5.2.6	Input ramp up/down time	67

8.5.3	Commands and events	67

8.5.3.1	Legend	67

8.5.3.2	Subscribe to V-Mixer control parameter changes	67

8.5.3.3	Input source connection	68

8.5.3.4	Set V-Mixer control parameter by remote application	68

8.5.3.5	Read V-Mixer control parameter by remote application	68

8.5.3.6	Notify remote application about V-Mixer control parameter change	68

8.6	V-Mode	69

8.6.1	Objects	69

8.6.2	Properties	69

8.6.2.1	Overview	69

8.6.2.2	Input source	69

8.6.2.3	Input source selector state	69

8.6.2.4	Audio transfer mode	70

8.6.3	Commands and events	70

8.6.3.1	Legend	70

8.6.3.2	Subscribe to V-Mode control parameter changes	70

8.6.3.3	Input source connection	70

8.6.3.4	Set V-Mode control parameter by remote application	71

8.6.3.5	Read V-Mode control parameter by remote application	71

8.6.3.6	Notify remote application about V-Mode control parameter change	71

8.7	Special provision for operations over UDP multicast	71

9.	Resource Discovery Protocol	73

9.1	Application	73

9.2	Resource data objects	73

9.3	Properties	73

9.3.1	ShowProfList: Show profile list	73

9.3.2	src_list: Source profile list	73

9.3.3	GpioList: GPIO resource list	74

9.3.4	console_hardware: Console hardware description	74

9.4	Resource discovery commands	75

9.4.1	Obtain a list of show profiles from console configuration data	75

9.4.2	Obtain a list of source profiles from console configuration data	75

9.4.3	Obtain a list of GPIO resources from GPIO unit configuration data	75

9.4.4	Obtain console hardware description from configuration data	76

9.5	Message sequence examples	76

10.	Unit Supervision	78

10.1	Application	78

10.2	Objects	78

10.2.1	Supervision entity	78

10.3 Properties	78

10.3.1	Overview	78

10.3.2	Device type	78

10.3.3	Software version ID	79

10.3.4	Supervision state	79

10.3.5	Controlling console IP address	79

10.3.6	Console type	80

10.3.7	Audio mixing mode	80

10.4	Message sequences	80

10.4.1	Overview	80

10.4.2	Unit status polling – the minimal response	81

10.4.3	Unit status polling – extended response	81

10.4.4	Reading an individual property	81

10.4.5	Reading bulk data	81

Appendix A.	Global status codes	82

Appendix B.	Source status codes used in Element	83

# 1.	Introduction

## 1.1	Scope

The first chapters of this specification define the common application-level protocol framework, to be universally applied to all application-specific areas of LW system control. After that a number of sub-protocol specifications follow.

The generic term "Livewire system control protocol" refers to the collection of all application-specific sub-protocols compliant to the common framework. It is not mandatory that each sub-protocol implements all features specified here. However, if a feature is implemented, it must be compliant to this specification.

The common application protocol framework definition includes the following:

- Common application notions, objects, identification and addressing methods

- Common message composition principles and ASCII-based syntax

- Universal data items, operation types and message sequences that would have identical meaning in all sub-protocols

- Requirements to the connection management and message transport services

The sub-protocol definitions include the following:

- Data items, operation types and message sequences that are specific only to certain narrow application areas, such as GPIO operations, automation system interface, etc. These features extend the capabilities defined in the common framework, but at the same time they must stay compliant to the universal rules.

## 1.2	Definitions and abbreviations

ID	Identifier

LW	Livewire

LWCP	Livewire control protocol

m/c	Multicast

PF	PathFinder application

p-p	Point-to-point

## 1.3	References

# 2.	Design goals and principles

## 2.1	Supporting concurrent carrier protocols

It will be possible in devices to support both the old CMsg/RUDP and the new ASCII-based carrier protocols at a time.

For that, we will assign new port numbers to the new Livewire system control protocol family. These ports will exist in parallel with the ports currently used by the old CMsg/RUDP based interfaces. Equivalent application content arriving over either of these carriers will cause equivalent application reactions.

This approach will ensure smooth replacing of the old CMsg/RUDP protocol with the new control interface family and compatibility between devices of different generations during the transition.

Application examples:

- Provide resource advertisement in two different formats in parallel - CMsg and ASCII

- Provide PF with access to GPIO services using the ASCII protocol, at the same time keeping the GPIO units compatible with the old SmartSurface interface as well

- Provide PF with access to SmartSurface show profile management services, still allowing SmartSurface to communicate with other components using the old CMsg based control interface

## 2.2	Future extensions and coexistence of protocol variants

In an environment where different entities may support different protocol versions and/or functional subsets, to be able to cooperate, entities need mechanisms ensuring the following:

- Exchange of information about the supported protocol versions and functional subsets

- Negotiation of the version/subset to be used in terms of a session

- Information about the syntax/format variants being used in an individual message

These mechanisms could be implemented using the following methods:

- One-way capability advertisement

- Interactive negotiation procedures

- Protocol version presentation in individual messages

- Format variant presentation in data units

Since we want to minimize duration of the specification process before starting application development, the first applications may appear implementing only limited multi-version support mechanisms. The original specification, with all its limitations, will be identified as version 1, and it will be used as the default if attempts to communicate in terms of a higher protocol or data version fail.

The further developments will observe and rely on the following rules:

- A new formal protocol version ID must be assigned, if the updated specification contains extensions or modifications, which may cause malfunction, if used together with the previous protocol version.

- The default protocol version and capability set always corresponds to the original specification. This applies to each individual sub-protocol

- Any enhanced protocol version or capability set must be negotiated, which has not been present in the default specification

- If negotiation is not requested or it fails, the default applies

## 2.3	Connections and transport services

All application transactions will be designed based on the following assumptions about the connection management and message transport services:

- The transport layer is capable to ensure reliable delivery, if required. It means that the application layer will not be required to comprise any reliability mechanisms within it. It will only request in each case a transport service of an adequate reliability class.

- The connection management and transport layers will support both point-to-point and broadcast/multicast types of communication. The application layer can select the desired delivery discipline but it does not need to comprise any mechanisms to implement these disciplines.

## 2.4	Universal message composition

Common message composition principles will apply universally to all sub-protocols of LWCP. They will determine

- Message structure elements and their sequence

- Object addressing methods

- Syntax rules

## 2.5	Unambiguous message interpretation

Every single message must be meaningful and unambiguously interpretable alone, regardless of the context.

This implicates that the operation name alone must determine the role of the message in terms of each sub-protocol. Using the same operation name for several roles concurrently, for example, for commands in one direction and responses in the other direction, is not acceptable - different operation names must be used for these purposes. This rule makes the protocol clear, symmetric and overall more robust.

Examples:

Correct:

Command:	READ <item name>

Response:	INDICATE <item name><value>

Incorrect:

Command:	SET <item><value>

Response:	SET <item><value>

## 2.6	Universal operation types

There will be a set of universal application messages, having common meaning and composition everywhere. At the outset this will include at least the messages to be used for common data item handling operations, like setting or reading values. As the protocol evolves, this set could be extended with more messages, TBD yet.

## 2.7	Application area specific items

As long as they stay compliant with the universal message composition rules, sub-protocols serving specific application areas may introduce their own dedicated operation types and data items, if they offer certain functional advantages over the defined common types.

Such approach seems to be the best tradeoff to combine flexibility and simplicity in each specific application area with protocol design that would be consistent throughout the system.

# 3.	General design

## 3.1	Application entities

There are application function groups residing in Surfaces, Engines, nodes, automation systems, PathFinder, which implement certain logic and communicate using LWCP messages.

## 3.2	Transport connections

Application entities use transport connections to communicate.

UDP/TCP port number 4010 is proposed as the default for all communications in terms of LWCP. This rule may have exceptions:

- The default port may be already in use by other user equipment. For this case a mechanism should exist for changing the default port number in field. It must not be available to the end user, only to the installation engineer. It could be implemented via a hidden low-level configuration data file, or a configuration web page requiring special access rights.

- Sub-protocols may be defined that would require their own dedicated ports. In this case new port numbers should be allocated from the range 4011 and higher.

There will be dedicated transport and connection management layers so that application entities do not need to implement any parts of these services. Application entities will have all the needed communication services at their disposal, and just will have to specify the required connection attributes.

Connection attributes:

- Connection class:

  - TCP connection (best-effort warranted delivery)

  - UDP unacknowledged datagram

- Distribution method

  - Point-to-point/multipoint (single or multiple destination addresses)

  - Broadcast (no concrete destination address, use multicast or broadcast)

- Application message aging

  - This does not apply to UDP. Each single message can be either delivered or lost immediately.

  - For TCP, at the theoretical level, this is an unresolved problem. There is no standard way to specify the max time for continuing attempts to deliver a message. For some application cases over-aged messages may cause significant system malfunction. A simple example is a PLAY command to an automation source. If the first command was delayed, the user will hit the button once again, which may result in unwanted skipping of a track, if both the original and the repeated messages are ultimately delivered. Te only answer to this, existing yet, is the following. We assume that all time-critical control transactions will happen inside a LAN, and that the probability is very low that a message could be significantly delayed on a LAN and still be delivered after some time. This could work practically, but it is not clean formally.

All combinations of the connection class and distribution method can be requested, except TCP connection with the broadcast distribution method, which is obviously impossible.

## 3.3	Objects and properties

Object is an origin or a target of an operation.

Objects may be simple or structured. Structured objects may be addressable at different structure levels – besides the identifier at the top level, each of the lower structure elements may have an ID too that can be used in messages.

An object may be, for example:

- Hardware resource: audio port, GPIO port

- Software resource: data unit, instance of an algorithm

- Thread: SmartSurface application control layer

- Complex multi-item structure: Collection of all resources assigned to a Livewire channel

Objects may have properties. The properties describe certain aspects of objects. A property may be, for example:

- Numeric parameter: audio signal level

- Logic state: state of a GPIO circuit

- Command: name of a batch program

Each property of an element of a complex structure is a property of the structure elements above it too, including the top-level object. Thus a single property may have several concurrent names that allow associating it with different structure levels.

## 3.4	Events

Events are significant occurrences in the system. These occurrences can be observed via object property changes.

An event can occur in the origin asynchronously, or it can be a side effect of received commands, or other external activities or conditions.

The essential difference between simple property changes and events is determined by their effect on the system operation. Example:

PROGRAM1 OUT_LEVEL=-24dB

PROGRAM1 silence_more_than_30_seconds=TRUE

The first is a simple property change, driving signal level displays. The second represents an emergency situation that requires immediate action to recover audio on air therefore it should be qualified as an event.

Examples of possible reactions to an event:

- Creating a log entry

- Sending a notification to other objects

- Sending commands to other objects

- Executing scripts

Existence of two rather different classes of property changes determines the need for differentiated notification mechanisms. Two different message types and procedures will be used for that:

- “Indication” message (INDI) for exposing data

- “Event” message (EVENT) as a signal and prompt for action

The INDI message does not impose any obligations on the receiver side, and it is never acknowledged. EVENT is an acknowledgeable message, and it may require that a receipt be returned and the due actions be taken by the recipient.

It is up to the application/script developer to make a decision in each particular case, what should be viewed as an event and what should stay a simple value update. Based on this decision, one or the other type of protocol messages and procedures should be used to report the property change.

Any arbitrary single, or a combination of multiple, property name-value pairs can be treated as an event. In the general case it would be reported using the EVENT message with the relevant name-value pairs in it.

Besides the method described above, dedicated properties may be defined for occurrences of any kind, including selected name-value pairs. These dedicated properties would directly reflect the states of the associated occurrences and take values “TRUE” or ”FALSE”. Meaningful occurrence names should be assigned to these properties, to obtain a clear and easy to use property set.

Although the protocol formally allows treating the dedicated per-occurrence properties as simple values as well, it is expected that in most practical applications they will be used to represent what is to be viewed as events, and each of these properties changing their states would trigger notifications.

## 3.5	Subscription to data flows

There will be a common discipline defined that would allow application entities to subscribe for receiving certain event notifications or property value indications. An individually addressed copy of each notification or indication will be sent to each subscriber currently on the list.

The subscription request message points to a certain object and its property. It instructs the source to add the client to the notification list associated with this property. In result the client will start receiving the requested event notifications or property value indications.

The subscription request does not determine the form of the notification. If the detected property change belongs to the event category, the event notification message will be used, otherwise a value indication message will be sent.

The subscription request also does not determine the conditions for generating notifications. They can be either associated with the property permanently, or, if such flexibility is needed, could be configured via the relevant application area sub-protocols. These conditions may include:

- Periodic value updates

- Value indication on change

- Taking special values or crossing thresholds

Thus the subscription request only determines a property to be observed, where the value indications are triggered according to the algorithm and conditions in the property owner.

The subscription service itself does not presume using any concrete connection class - TCP or UDP. The latter should depend rather on the role of the data flow and the effect of delivery failures on the system operation.

The subscription mechanism does not need to be involved to receive data flows that are broadcast by the origin.

## 3.6	Livewire channels

Livewire channels are virtual pipes that exist in Livewire broadcast, at the application level. While physically the control events are broadcast, these pipes allow application entities to selectively process only certain flows of them, rather than fully processing all.

If a channel is allocated for exclusive use by two application entities, then from the application logic viewpoint it serves as a virtual point-to-point connection between them.

The existing (CMsg/RUDP) GPIO control interface could be an example for this type of communication. Physically all GPIO events in the system are broadcast on a common m/c address. At the application level each GPIO port owns a channel number, which is thus exclusively reserved for signaling between SmartSurface and that single GPIO Port.

With migration to TCP, the channels would be obviously no more needed to implement p-p configuration at the device level, still they may appear useful to organize virtual pipes inside a TCP link, like described above.

In LWCP messages the channel may appear in the role of a “super-object” that represents the whole collection of items hooked on that channel.

## 3.7	Access control

RQ 4176

### 3.7.1	General logic

Application entities may restrict access to their resources by allowing only certain authenticated clients to execute protected operations. Generally the complexity of such restrictions may range from all operations unprotected to detailed mapping of multiple access levels to individual operation types and data sets.

The actual access level is an attribute of the connection established to the resource. After opening a connection, the default access level will be set automatically.

Further the client may continue working at the default level, or it may request an access level change by means of a login procedure. The access level change may be requested at any time during the connection, and repeatedly as well.

If a login message is received without the user name/password pair, or with an invalid pair, the connection would stay at, or return to the default access level. Opening a new connection, even for the same client, will reset the access level to the default as well.

In a system supporting differentiated access levels, the client can explicitly specify the preferred level, using the ACCLEV property of the LOGIN request.

To ensure smooth upgrading from older software versions that may implement unprotected full access, the protected application entities must offer a configuration option to completely disable the protection mechanism. By leaving the password blank, or clearing it, the protection will be deactivated, and full access rights will be automatically granted to all client connections.

Since there seems to be no practical value in requiring the password from local clients connecting via 127.0.0.1, and also to stay consistent with the specified behavior of LWRP interface, these connections are automatically granted full access rights, without the login procedure.

### 3.7.2	09 Jul 2008:   The current requirements

The following statements define the immediately required functionality. It complies to the general logic as described above, but represents a narrowed subset of it, with the needed details provided.

- Two access levels are defined:

  - “Read-only”:   The client can only read data values and receive notifications (including using the subscription service if such is offered), but can not execute any commands changing the operation states or configuration of the unit. Presently, from the group of the universal operations, the SET command is the only one that is not allowed at this level.

  - “Full control”:   The client can execute all commands, including SET.

- If a password is set, the password protection is activated, and the default access level for all connections is “read-only”. The “read-only” access is set immediately after opening every new connection, before login, and it would be maintained in the case of unsuccessful login attempts.

- After a client has provided a LOGIN message with any valid user name/password pair, its connection is granted the “full control” access level. Login messages without the user name/password pair, or containing an invalid pair, or generally incomplete or corrupted, do not change the access level to “full control” and leave it at “read-only”. The “full control” access level will not be transferred from the previous connection, even for the same client - every new connection requires a new login.

- Once granted, the “full-control” access lasts until either:

  - the connection is closed

  - a LOGIN message with an empty or invalid name/password pair is received that would reset the access level to “read-only”.

- If no password is set, the password protection is deactivated, and the access level is permanently “full control” for all active connections.

- On the devices supporting local clients via 127.0.0.1, these connections are automatically granted the “full control” access level, without the login procedure.

- The resource must accept commands without the access level property ACCLEV.

- The resource is not required to support the access level property ACCLEV, and clients must not rely on having such support in the resource, unless including it into certain resources has been explicitly agreed between developers.

- The resource is not required to provide, and clients must not rely on receiving any status responses to their login requests, neither successful nor unsuccessful, unless including such support into certain resources has been explicitly agreed between developers.

# 4.	Message composition

## 4.1	Character set

All messages consist exclusively of printable ASCII characters, codes 32 to 126 (decimal).

## 4.2	Case sensitivity

v.12

- Message parsing is case-insensitive. That means – all names, which identify LWCP operations, objects, properties, enumerated values, system items, will be recognized by the receiver regardless of the actual character cases in the message.

- Protocol is transparent for text data carried as property values. That means – character cases in text property values can not be modified during transportation or storage.

## 4.3	Special symbols

Message structure element separator: Space (SP)

PLAY CD1

Object entity identifier separator: Number sign

CH#1001

GPO#ON_AIR_LAMP

Object structure level separator: Full stop

CH#1001.OUT#2

Property value separator: Equals sign

STATE=HIGH

List separator: Comma. Inserting an arbitrary number of the “Space” symbols is allowed both before and after the comma.

5.25,10.1,–27.3

5.25, 10.1  ,  –27.3

STATE=HIGH, TIME=250

Literal constant delimiters: Quotation marks

“Mike’s Morning”

Value list/array delimiters: Square brackets

[70,2.0,BANDPASS]

[-50,-40,-30,-20]

System item name discriminator: Dollar sign

$DebugMode

## 4.4	Names

Names are used to identify:

- Operation types

- Object classes

- Object entities

- Properties

- Enumerated constants

- System items

Name is an ASCII string, beginning with a letter, and consisting of digits, letters, and “low line” characters (“_”).

Names of special-purpose system items follow the same rule, but they always have a prefix – a dollar sign (“$”) as the discriminator.

Examples:

PLAY

AUDIO_EVENT_3

$DebugMode	- This one is a system name

## 4.5	Property values

A value of a property can be one of the following:

- Literal constant

- Numeric constant

- Named enumerated constant

- List/array of constants

- Encapsulated data

An arbitrary ASCII string enclosed in quotation marks must be interpreted as a literal constant:

“Mike’s Morning”

Line breaks are allowed in literal constants, encoded as “/n”:

“Select/nsource”

An ASCII string beginning with a decimal digit and not enclosed in quotation marks must be interpreted as a numeric constant:

5.25

An ASCII string beginning with a letter and not enclosed in quotation marks must be interpreted as a named enumerated constant:

HIGH

A number of constants of any type, separated by commas, and enclosed in square brackets, form a list/array of constants:

[70,2.0,BANDPASS,16000,3.0,SHELF]

[-50,-40,-30,-20]

Nested pairs of brackets denote nested lists. Multidimensional arrays are a special case of nested lists.

[[-50,GREEN],[-12,YELLOW],[-3,RED],5,200]

[[1,0,0,0],[0,1,0,0],[0,0,1,0]]

An arbitrary sequence of ASCII characters enclosed between the encapsulation begin-end markers must be interpreted as encapsulated data:

%BeginEncap%Data go here%EndEncap%

## 4.6	Numeric data presentation

All types of numeric data are encoded as ASCII strings.

Decimal integers:

5	1000	-27

Decimal fixed point:

5.25	10.1	–27.3

Decimal floating point

5e12	10.1e-20	-27.3e-20

This form is considered rarely needed. Implementation is optional, according to a use convention.

Hex:

0xA	0x100	0xBE75

Binary

0b101	0b11001100

## 4.7	Encapsulated data

The following tags mark beginning and end of an encapsulated ASCII data block:

%BeginEncap%

%EndEncap%

The enclosed data block is not allowed to comprise the end marker as a sub-string. Except this restriction, it is allowed to contain arbitrary printable ASCII data.

Note:	There was an alternative proposal too, to use a non-printable character for the end marker. It would formally solve the content restriction problem, but could appear less convenient in use, due to presence of invisible characters in the message body. Possibly, incompatibility with some carrier protocols could be encountered too. Therefore using printable tags is being proposed, based on the following considerations:

- This is not a general-use data transport protocol, therefore we can assume that the encapsulated data in most cases would contain constructs defined in terms of Livewire protocol family design

- The proposed tags are relatively long and “exotic”, and they begin with a special character that usually does not precede letters. This makes the probability extremely low that the enclosed data content could mimic the delimiters by accident.

- If practical applications discover problems with these printable delimiters, we have the door open to fix this by defining an extended protocol version, where using alternative delimiters would be allowed. These new delimiters might rely on using non-printable characters, or any other appropriate methods.

## 4.8	Message delimitation

Each application message is a line of ASCII characters terminated by CR+LF.

It is allowed to pack several CR+LF terminated application messages into a single transport service request. From the application logic viewpoint such bundled messages would be handled exactly as a series of equivalent single messages.

This protocol allows that CR+LF appear in encapsulated data in the message body, as long as using the encapsulation begin-end markers is strictly observed. This design relies on a restriction that an application message would never be split over several outgoing transport service requests. It means that the incoming transport service would always deliver the whole application message (or several messages), never a fragment. Since the transport services ensure application message integrity, interpretation errors should not occur.

## 4.9	Message structure

The general structure is common for all message types.

At the top level a message may contain the following elements, and they follow in a fixed sequence as shown below, separated by spaces:

<Operation> SP <Object> SP <Property list> SP <System items>

Operation identifies the activity being requested or reported by the message. Most of them will be either a command, a response to a command, a data or status indication, or an event report, but possibly some other classes could be distinguished too. Operation is the only mandatory element in all messages.

Object identifies the target or the origin of the operation, unless it is determined by another method. There may be one object in a message, or none.

The properties always belong to the specified object. List of properties cannot be included if there is no object specified. From the message structure viewpoint, the list of properties is open. It may contain an arbitrary number of items, or be empty. The operation and object type will determine if each concrete property is mandatory or optional. The list of properties for any object may be extended as system evolves.

System items are optional and may be present in all types of messages. They are intended to serve system and protocol control needs and are not necessarily related to end-application objects. They will be unconditionally distinguishable thanks to their tags, all beginning with a special prefix – a character sequence reserved exclusively for this use. This allows placing them at the end of any message, regardless of presence or absence of other message elements.

## 4.10	Construction of message elements

### 4.10.1	Operation

Operation is a simple item with no nested sub-structure and no value. It is identified by a name.

The length of operation name may be arbitrary. Not restricting all operation names to one common length has the advantage that an intuitive, easy to read and remember name can be assigned to each operation type, thus improving readability and minimizing the probability of misuse and misinterpretation. The latter is the weak point of using too short abbreviation-style names.

Examples:

PLAY

SET

SELECT

### 4.10.2	Object

Object may be a simple or a structured item, and several entities can exist at each structure level. A full object identifier may be composed of class names and entity identifiers.

Class name is a name. There is a mandatory class name at each structure level, at least one.

Entity identifier is a decimal positive integer number or a name. Entity identifier must be included if several numbered or enumerated entities exist at that structure level.

Examples of the possible forms of the object identifier:

- Single structure level without entity ID. This is the simplest form of the object, and it is identified by a name:

CONFIG_DATA

- Multi-level nested structure without entity ID:

CONFIG_DATA.SYSTEM_SETTINGS

- Single structure level with entity ID:

CHANNEL#1001

SCRIPT#ACTIVATE_BACKUP

- Multi-level nested structure with entity IDs:

CHANNEL#1001.GPI#7

CHANNEL#1001.GPO#ON_AIR_LAMP

### 4.10.3	Property list

Each property is a simple item with no nested sub-structure. It is identified by a name.

A property may have several concurrent names that make possible associating it with different object structure levels. This principle allows the application developer to choose such form for pointing to a property, which seems most convenient and intuitive in the given context. The following could be an example of such equivalents:

CHANNEL#1001.GPI#7 STATE

CHANNEL#1001 STATE_OF_GPI7

Depending on the message context, the property list may consist of either

- Property name-value pairs

- Property names only

Property list examples:

Two property name-value pairs on the list:

SET CH#1002.GPO#7 STATE=HIGH,TIME=250

Single property on the list, name only:

GET CH#1001.GPI#2 STATE

Single property on the list:

EVENT AIN#5 CLIP=TRUE

### 4.10.4	System items

System items may serve rather different and difficult to predict purposes, therefore their composition is restricted to a less extent than other message elements. The name of a system item determines its role in the operation and the meaning of the data following it. It is the application developer’s responsibility to avoid abuse, and keep these items as simple as possible.

# 5.	Universal operations

## 5.1	Universal operations summary

The following operations are defined for common use throughout the system and are to be interpreted and processed in compliance with descriptions given below.

| Operation name | Meaning |
| --- | --- |
| LOGIN | Login request |
| SET | Request to set property value |
| GET | Request to indicate property value |
| INDI | Property value indication |
| EVENT | Event notification |
| SUB | Subscribe to data flow |
| UNSUB | Unsubscribe from data flow |
| ACK | Universal acknowledgement |

## 5.2	Message descriptions

### 5.2.1	LOGIN: Login request

RQ 4176

| Description | The LOGIN message is used to request a certain access level to a certain entity. The request message points to an object that requires passing the access control, and it may carry a user name and password pair as parameters. A login message without the user name/password pair can be used to reset the access rights to the automatic default. Optionally the message may contain an access level parameter: - If the object does not support differentiated access levels, this parameter will be ignored - If the object supports differentiated access levels, and this parameter is included, the user attributes will be checked against the requested access level exactly. - If the object supports differentiated access levels, and this parameter is not included, the user attributes will be checked and the lowest access level from the range associated with that particular user will be granted by default. Optionally, acknowledgement can be requested by means of including the $ACK item, in which case a transaction identifier $TRXI is mandatory. Otherwise the transaction identifier is optional. |
| --- | --- |
| Operation name | LOGIN |
| Message elements | Object identifier – mandatory USER and PWD properties – mandatory to authenticate a user, or not included to reset the access level ACCLEV property – optional $ACK – optional, force acknowledgement $TRXI – conditional |
| Examples v.09 | Basic login request: LOGIN CONFIG USER=”MIKE”,PWD=”AXIA” Basic login request, ack required: LOGIN CONFIG USER=”MIKE”,PWD=”AXIA” $ACK $TRXI=555 Login request to a certain access level: LOGIN CONFIG USER=”MIKE”,PWD=”AXIA”,ACCLEV=2 $ACK $TRXI=555 |

### 5.2.2	SET: Set value

| Description | The SET message is used to set a value of a property. The request message must contain the identifier of the object-owner and one or multiple property values or name-value pairs. Optionally, a general acknowledgement can be requested by means of including the $ACK item, or indication of value after execution by means of the $IND item. In the latter cases a transaction identifier $TRXI is mandatory. Otherwise the transaction identifier is optional. Returning an ACK message with a proper status code is mandatory, if the command structure or some of the included items appear invalid, or setting the new value fails due to another reason. |
| --- | --- |
| Operation name | SET |
| Message elements | Object identifier – mandatory Property list with values – mandatory $ACK – optional, force acknowledgement $IND – optional, force indication $TRXI – conditional |
| Examples | SET CH#1001.GPO#2 STATE=HIGH,TIME=250 – no response requested SET CH#1002.GPO#5 STATE=LOW $ACK $TRXI=555 – ACK is requested SET CH#1002.GPO#5 STATE=LOW $IND $TRXI=555 – INDI is requested |

### 5.2.3	GET: Read value

| Description | The GET message is used to request indication of the value of a property. A response message INDI is required to be returned with the actual value. A status code may be included in the response, if reading the value fails, to indicate the reason of the failure. An ACK message with a proper status code must be returned instead of INDI, if the command structure or some of the included items appear invalid. The request message must contain the identifier of the object-owner and one or multiple property names without values. Optionally, a transaction identifier $TRXI may be included. |
| --- | --- |
| Operation name | GET |
| Message elements | Object identifier – mandatory Property list, names only – mandatory $TRXI – optional |
| Examples | GET AOUT#2 PEAK,RMS GET AIN#4 PSNM,LWSE,INGN $TRXI=555 |

### 5.2.4	INDI: Indicate value

| Description | The INDI message is used to indicate the value of a property. This message may be sent in response to a SET $IND command, a read request GET, or at any other instant, according to the operation logic of the property owner, for example – periodic updates of a meter or sensor value. A status code may be included to report special conditions. The indication message must contain the identifier of the object-owner and one or multiple property values or name-value pairs. Value is not included if reading fails, or target of the request was invalid. If an INDI message is generated in response to a SET or GET message, and the request message includes a transaction identifier $TRXI, the latter must be copied into the response message. If the property owner generates the INDI message without a prior request, including the transaction identifier is optional. |
| --- | --- |
| Operation name | INDI |
| Message elements | Object identifier – mandatory Property list with/without values – mandatory $STATUS – conditional $TRXI – conditional |
| Examples | INDI CH#1001.GPI#2 STATE=HIGH INDI AOUT#2 PEAK=6.4,RMS=-2.1 INDI AOUT#2 PUUK $STATUS=8 $TRXI=555 - unknown property name |

### 5.2.5	EVENT: Event notification

| Description | The EVENT message is used to send an asynchronous notification about an event. Optionally, acknowledgement can be requested by means of including the $ACK item, in which case a transaction identifier $TRXI is mandatory. Otherwise the transaction identifier is optional. The notification message must contain the identifier of the object-origin and one or multiple property name-value pairs, which sufficiently describe the event. Returning an ACK message with a proper status code is mandatory, if the command structure or some of the included items appear invalid, or the required reaction to the event cannot be ensured due to another reason. |
| --- | --- |
| Operation name | EVENT |
| Message elements | Object identifier – mandatory Property list with values – mandatory $ACK – optional, force acknowledgement $TRXI – conditional |
| Examples | Selected combination of values is handled as an event: EVENT AIN#5 LEVEL=-60.0,TIME=10 A dedicated per-event property changes its state: EVENT AIN#2 CLIP=TRUE $ACK $TRXI=555 |

### 5.2.6	SUB: Subscribe to data flow

| Description | The SUB message is used to subscribe to a data flow. The request message points to a certain object and its property. It instructs the source to add the client to the notification list associated with this property. In result the client will start receiving the requested event notifications or property value indications. The default is subscription to all properties associated with the object – in this case the property list is not required. Optionally, acknowledgement can be requested by means of including the $ACK item, in which case a transaction identifier $TRXI is mandatory. Otherwise the transaction identifier is optional. |
| --- | --- |
| Operation name | SUB |
| Message elements | Object identifier – mandatory Property name list – optional, default is all properties $ACK – optional, force acknowledgement $TRXI – conditional |
| Examples | Subscribing to selected events, ACK requested in the first example: SUB AOUT#2 CLIP,NOCLIP $ACK $TRXI=555 SUB GPI#2 STATE Subscribing to all properties associated with the object: SUB AOUT#2 Subscribing to simple property value updates: SUB AIN#5 PEEK,RMS |

### 5.2.7	UNSUB: Unsubscribe from data flow

| Description | The UNSUB message is used to unsubscribe from a data flow. The request message points to a certain object and its property, and it instructs the source to remove the client from the notification list associated with this property. In result the client will stop receiving property value indications or event notifications. The default is unsubscribing from all notifications associated with the object, including simple value updates – in this case there is no property list in the message. Note that this is different from the subscription request, where the default applies only to events, not the simple value updates. Optionally, acknowledgement can be requested by means of including the $ACK item, in which case a transaction identifier $TRXI is mandatory. Otherwise the transaction identifier is optional. |
| --- | --- |
| Operation name | UNSUB |
| Message elements | Object identifier – mandatory Property name list – conditional $ACK – optional, force acknowledgement $TRXI – conditional |
| Examples | Unsubscribing from a selected event: UNSUB AOUT#2 CLIP,NOCLIP $ACK $TRXI=555 UNSUB GPI#2 STATE Unsubscribing from all notifications: UNSUB AOUT#2 Unsubscribing from a property value: UNSUB AIN#5 PEEK,RMS |

### 5.2.8	ACK: Acknowledgement

| Description | The ACK message is used to acknowledge either reception or execution of the command, according to the logic defined for the recipient. Rather than simple confirmation of delivery ensured by TCP, this response provides feedback at the application level. It informs the origin of the request whether the message could be interpreted and/or executed. This is the universal form of acknowledgement, to be used by default, unless the concrete procedure requires another specific type of response. If the command message includes the $ACK item, returning this response message is mandatory. The default status code is “OK”, and it is allowed to be not included. In all other cases the status code is a mandatory element of ACK messages. Object ID must be included if there is no other mechanism associating the ACK message with the origin of the operation. Object ID may be included also to assist selective processing in the case of broadcast type of communications. If the request message of any type includes a transaction identifier, the latter must be copied into the ACK message. |
| --- | --- |
| Operation name | ACK |
| Message elements | Object identifier – conditional $STATUS – conditional $TRXI – conditional |
| Examples | ACK – OK, it is the default code ACK $STATUS=OK – OK, same as above, but status code is included ACK CH#1001 $STATUS=0 ACK $STATUS=2 $TRXI=555 |

## 5.3	Universal objects

### 5.3.1	UNIT: The root object

RQ 4176

This is the formal root of all application related resources and aspects existing in a concrete unit. It is being introduced to obtain the means for addressing the application as a whole, instead of talking to individual resource groups or aspects. The class name for it is:

unit

The first, and the only defined so far, use for this super-object is the login procedure. Originally all Livewire devices will only support one common password for all groups of application operations. Therefore, to keep the interface logic clean, a login target is needed that covers it all.

login unit user=”Mike”, pwd=”Axia”

In the case a mechanism for selective login to smaller function groups would be required later, a corresponding collection of object names would be introduced to represent certain parts or aspects of the unit. These names may identify interim layers, referencing whole function groups, as well as they may go down till concrete objects being used as targets in control operations. General examples (not to be used as final name definitions):

login config_data			– accessing all configuration data stored in the unit

login config_data.sources		– accessing only source configuration data stored in the unit

login remote_operations		– accessing the entire remote operations aspect

login remote_operations.vmix	– accessing the V-mixer operations only

## 5.4	Universal properties

### 5.4.1	USER: User name for login

| Description | Mandatory item in the LOGIN message - identifies the user attempting to login. If the resource supports differentiated access levels, the user name determines the allowed range. |
| --- | --- |
| Item name | USER |
| Data type | Literal constant |
| Value range | No content or length limitations are defined by this protocol. The resource may introduce such limitations though. |

### 5.4.2	PWD: Password for login

| Description | Mandatory item in the LOGIN message, always used in pair with the USER item. It confirms that an authorized person is attempting to login. |
| --- | --- |
| Item name | PWD |
| Data type | Literal constant |
| Value range | No content or length limitations are defined by this protocol. The resource may introduce such limitations though. |

### 5.4.3	ACCLEV: Access level for login

| Description | Optional item in the LOGIN message. It may be used if the resource supports differentiated access rights levels. |
| --- | --- |
| Item name | ACCLEV |
| Data type | Numeric constant Named enumerated constant |
| Value range | Grouping of access rights may depend on the concrete application area, therefore no range restrictions or value interpretations are defined at this universal level. The relevant subprotocol must define them in cases, where differentiation of access rights is implemented. |

## 5.5	Universal system items

### 5.5.1	$PROTID: Protocol version identifier

| Description | The protocol version ID is a reference to the interface specification variant describing the expected behavior and rules for composing and interpreting messages. This item is not required but is allowed to be present in all messages. If it is not present, the session default applies, or if no session default is established, the global default – protocol version 1 applies. Some data items may have internal structure and packing variants allowed in terms of a single protocol version. Such variants must be identifiable by means of internal elements of these data items. In this case the protocol version must be a sufficient key to locate and interpret the internal format descriptor or its equivalent. |
| --- | --- |
| Item name | $PROTID |
| Data type | Numeric constant |
| Value range | Positive integer decimal number |

### 5.5.2	$MSGI: Message identifier

| Description | Message identifiers may be used to personalize individual messages. This item is not required but is allowed to be present in all messages. It should be used if the ability to recognize individual messages is required by the operation logic. The identifier value is assigned by the sender, and it must be ensured that this value is unique among all incoming messages of all potential receivers. Thus for isolated p-p sessions uniqueness must be ensured only in terms of the current session, whereas for messages being distributed using the broadcast method, the identifiers must be unique system-wide. The latter imposes that some kind of unit ID were included as a part of the broadcast message identifier. |
| --- | --- |
| Item name | $MSGI |
| Data type | Numeric constant Literal constant |
| Value range | - Uniqueness must be ensured as stated above - Numeric identifiers are limited to positive integer decimal or hexadecimal numbers Examples of numeric identifiers: $MSGI=87342 $MSGI=0x5EAB65AA6 Examples of literal identifiers: $MSGI=”87342” $MSGI=”192.168.10.101.9538” |

### 5.5.3	$TRXI: Transaction identifier

| Description | Transaction identifiers may be used to link related messages in terms of complex operations. This item is not required but is allowed to be present in all messages. Exclusively the origin of the transaction decides if the identifier will be used or not. If it was included at the origin, all the following messages exchanged in terms of this transaction must contain the same identifier value as a reference. If it was not included, the following messages must not contain it either. The identifier value is assigned by the origin, and it must be ensured that this value is unique among all transactions that may be currently active on all potential receivers of messages being exchanged in terms of this transaction. |
| --- | --- |
| Item name | $TRXI |
| Data type | Numeric constant Literal constant |
| Value range | - Uniqueness must be ensured as stated above - Numeric identifiers are limited to positive integer decimal or hexadecimal numbers Examples of numeric identifiers: $TRXI=87342 $TRXI=0x5EAB65AA6 Examples of literal identifiers: $TRXI=”87342” $TRXI=”192.168.10.101.9538” |

### 5.5.4	$STATUS: Status code

| Description | Status code is allowed to be included in all response messages to report execution status or special conditions. It is a mandatory item in the universal acknowledgement messages ACK, except the convention about “OK” as the default status. |
| --- | --- |
| Item name | $STATUS |
| Data type | Numeric constant Named constant |
| Value range | Enumerated set of codes. See Appendix A for global status codes, which are common for all application areas. Besides these, local codes may be defined in terms of application-specific sub-protocols. Examples of numeric codes: $STATUS=0 – OK $STATUS=2 – Value out of range Examples of named codes: $STATUS=OK $STATUS=OUT_OF_RANGE |

### 5.5.5	$ACK: Force acknowledgement

| Description | This item is allowed to be included in all command messages, which do not require other specific response types. If it is included, the receiver must acknowledge receiving and execution of the command. |
| --- | --- |
| Item name | $ACK |
| Data type | This item is a complete instruction rather than a variable. Presence of the item name alone fully determines meaning, and no value data type is associated with it. |
| Value range | No value |

### 5.5.6	$IND: Force indication

| Description | This item is allowed to be included in SET command messages. If it is included, the receiver must respond with indicating the new property values after execution of the command. |
| --- | --- |
| Item name | $IND |
| Data type | This item is a complete instruction rather than a variable. Presence of the item name alone fully determines meaning, and no value data type is associated with it. |
| Value range | No value |

## 5.6	Message sequence examples

### 5.6.1	Notation

All object and property names and values in this section are for illustration only. They must not be viewed as a sub-protocol specification, although some names may match incidentally.

Abbreviations:

C	Client

S	Server

### 5.6.2	Login

RQ 4176	v.10a

#### 5.6.2.1	Unprotected interface

No password is set in the unit, to disable the protection

C: Open connection to the unit

S: (granted “full control” rights)

...

Any LOGIN message, including an incomplete one, corrupted, or containing an invalid name/password pair

C: LOGIN UNIT USER=”MIKE”,PWD=”BBBB”

S: (retained “full-control” rights)

#### 5.6.2.2	Protected interface, successful login, closing the connection resets the rights

Password is set in the unit, and protection is enabled

C: Open connection to the unit

S: (granted “read-only” rights)

...

Valid pair of user name and password is provided

C: LOGIN UNIT USER=”MIKE”,PWD=”AXIA”

S: (granted “full-control” rights)

...

Closing the connection resets the rights

C: Close the connection

...

C: The same client reopens the connection

S: (granted “read-only” rights)

#### 5.6.2.3	Protected interface, repeated login procedures during a single connection

Password is set in the unit, and protection is enabled

C: Open connection to the unit

S: (granted “read-only” rights)

...

Incomplete LOGIN message received

C: LOGIN

S: (retained “read-only” rights)

...

Invalid pair of user name and password is provided

C: LOGIN UNIT USER=”MIKE”,PWD=”BBBB”

S: (retained “read-only” rights)

...

Valid pair of user name and password is provided

C: LOGIN UNIT USER=”MIKE”,PWD=”AXIA”

S: (granted “full-control” rights)

...

LOGIN message without any name/password pair

C: LOGIN UNIT

S: (reset to “read-only” rights)

#### 5.6.2.4	Unsupported access level parameter is ignored

Password is set in the unit, and protection is enabled

C: Open connection to the unit

S: (granted “read-only” rights)

...

Valid pair of user name and password is provided. The unsupported preferred access level property will be 	ignored, and the rest of the command will be executed.

C: LOGIN UNIT USER=”MIKE”,PWD=”AXIA”,ACCLEV=5

S: (granted “full-control” rights)

#### 5.6.2.5	Local client always has the full rights

Password is set in the unit, and the external interface is protected. A local client is connecting.

C: Open connection to the unit via 127.0.0.1

S: (granted “full control” rights)

...

LOGIN messages do not affect the access level

C: LOGIN UNIT

S: (retained “full-control” rights)

#### 5.6.2.6	Status responses

09 Jul 2008:   This may be implemented according to a local convention, but is not universally required

Basic login request, response is requested, successful attempt.

C: LOGIN UNIT USER=”MIKE”,PWD=”AXIA” $ACK $TRXI=555

S: ACK $TRXI=555 $STATUS=OK

Basic login request, no response requested, unsuccessful attempt.

C: LOGIN CONFIG USER=”MIKE”,PWD=”BBBB”

S: ACK $STATUS=AUTHORIZATION_FAILURE

### 5.6.3	Set property value

1) Success, no response requested

C: SET GPO#2 STATE=HIGH

S:

2) Success, ACK is requested

C: SET GPO#2 STATE=HIGH $ACK $TRXI=555

S: ACK $TRXI=555

3) Success, INDI is requested

C: SET GPO#2 STATE=HIGH $IND $TRXI=555

S: INDI GPO#2 STATE=HIGH $TRXI=555

4) Failure, can’t set value. In the second and third examples, although INDI was not requested, in the case of an execution failure the owner uses it as the more informative form of response.

C: SET AIN#2 LWSE=DISABLE $IND $TRXI=555

S: INDI AIN#2 LWSE=ENABLE $TRXI=555 $STATUS=AUTHORIZATION_FAILURE

- OR -

C: SET AIN#2 LWSE=DISABLE $ACK $TRXI=555

S: INDI AIN#2 LWSE=ENABLE $TRXI=555 $STATUS=AUTHORIZATION_FAILURE

- OR -

C: SET AIN#2 LWSE=DISABLE

S: INDI AIN#2 LWSE=ENABLE $STATUS=AUTHORIZATION_FAILURE

5) Failure, invalid value. In the second and third examples, although INDI was not requested, in the case of an execution failure the owner uses it as the more informative form of response.

C: SET GPO#2 STATE=HIHH $IND $TRXI=555

S: INDI GPO#2 STATE=LOW $TRXI=555 $STATUS=INVALID_VALUE

- OR -

C: SET GPO#2 STATE=HIHH $ACK $TRXI=555

S: INDI GPO#2 STATE=LOW $TRXI=555 $STATUS=INVALID_VALUE

- OR -

C: SET GPO#2 STATE=HIHH

S: INDI GPO#2 STATE=LOW $STATUS=INVALID_VALUE

6) Failure, unknown property name. Responding with an INDI message would be rather confusing than useful therefore ACK is used.

C: SET GPO#2 STAAT=HIGH $TRXI=555

S: ACK $TRXI=555 $STATUS=UNKNOWN_PROPERTY

7) Failure, unknown object. Responding with an INDI message would be rather confusing than useful therefore ACK is used.

C: SET GPU#2 STATE=HIGH $TRXI=555

S: ACK $TRXI=555 $STATUS=UNKNOWN_OBJECT

### 5.6.4	Read property value

1) Success

C: GET AOUT#4 PEAK

S: INDI AOUT#4 PEAK=[2.4,2.7]

2) Failure, unknown property name

C: GET AOUT#4 PUUK $TRXI=555

S: ACK $TRXI=555 $STATUS=UNKNOWN_PROPERTY

### 5.6.5	Indicate property value

1) Owner indicates a property value upon an internal condition

C:

S: INDI AOUT#4 PEAK=[2.4,2.7],RMS=[1.2,0.7]

### 5.6.6	Broadcast event notification

Broadcast notifications do not require subscription.

1) Change of an “event” property occurred; acknowledgement is not requested.

S: EVENT AOUT#4 CLIP=TRUE

C:

2) Property value change reported as an event; acknowledgement is requested; channel number is included in the response to assist selective processing at m/c receivers.

S: EVENT CH#1001.GPI#2 STATE=HIGH $ACK $TRXI=555

C: ACK CH#1001 $TRXI=555

### 5.6.7	Subscribe to data flow

1) Observing a single property, each of two values handled as an event

Signal in the normal range

S:

C:

Subscribe to clipping threshold notifications

C: SUB AOUT#2 CLIP

S:

Detected clipping

S: EVENT AOUT#2 CLIP=TRUE

C:

Signal returned into the normal range

S: EVENT AOUT#2 CLIP=FALSE

C:

2) Observing two properties, the “TRUE” value of each is handled as an event

Signal in the normal range

S:

C:

Subscribe to clipping threshold notifications

C: SUB AOUT#2 CLIP,NOCLIP

S:

Detected clipping

S: EVENT AOUT#2 CLIP=TRUE

C:

Signal returned into the normal range

S: EVENT AOUT#2 NOCLIP=TRUE

C:

3) Observing a property value, events are associated with it, acknowledgements requested

Detected state change

S:

C:

Subscribe to GPIO input state notifications

C: SUB GPI#2 STATE $ACK $TRXI=555

S: ACK $TRXI=555

Detected state change

S: EVENT GPI#2 STATE=HIGH $ACK $TRXI=555

C: ACK $TRXI=555

Detected state change

S: EVENT GPI#2 STATE=LOW $ACK $TRXI=555

C: ACK $TRXI=555

4) Observing a property value, no events associated with it, source generates periodic refresh messages

Meter data refresh interval

S:

C:

Subscribe to level meter data flow

C: SUB AOUT#4 PEAK,RMS $ACK $TRXI=555

S: ACK $TRXI=555

Meter data refresh interval

S: INDI AOUT#4 PEAK=[2.4,2.7],RMS=[1.2,0.7]

C:

Meter data refresh interval

S: INDI AOUT#4 PEAK=[2.8,2.6],RMS=[1.1,0.8]

C:

### 5.6.8	Unsubscribe from data flow

1) Stop observing all events and properties

Detected clipping

S: EVENT AOUT#2 CLIP=TRUE

C:

Unsubscribe from all notifications

C: UNSUB AOUT#2 $ACK $TRXI=555

S: ACK $TRXI=555

AOUT#2 - any property change

S:

C:

2) Stop observing a single property

Meter data refresh interval

S: INDI AOUT#2 RMS=-24

C:

Unsubscribe from audio level updates

C: UNSUB AOUT#2 RMS $ACK $TRXI=555

S: ACK $TRXI=555

Meter data refresh interval

S:

C:

Detected clipping

S: EVENT AOUT#2 CLIP=TRUE

C:

# 6.	GPIO Application Protocol

## 6.1	Application

The GPIO protocol is intended to carry GPIO circuit state related data and commands to and from GPIO resources, both implemented in physical hardware and emulated by software.

## 6.2	GPIO application objects

- Objects of the GPIO application are:

- Logic input circuits

- Logic output circuits

- Each logic circuit is assigned a LW channel number, and the LW channel number is the exclusive top-level entry to the circuit.

- Each logic circuit has a unique entity identifier in terms of the whole object collection represented by the LW channel. The LW channel is viewed as a super-object.

- The class name for LW channels is:

CH

- The entity identifier is a positive integer decimal number, thus the object identifier for the resource collection represented by LW channel is:

CH#<channel number>

- The class names for logic input and output circuits are respectively:

GPI

GPO

- The entity identifier is a positive integer decimal number, thus the object identifiers for logic input and output circuits are respectively:

GPI#<circuit number>

GPO#<circuit number>

- Full references to an input and output circuits:

CH#<channel number>.GPI#<circuit number>

CH#<channel number>.GPO#<circuit number>

- Each input circuit has a property:

STATE		- read-only

- Each output circuit has the following properties:

STATE		- read-only

PULSE		- write-only

## 6.3	Properties

### 6.3.1	GPI STATE: State of input circuit

| Description | The STATE property of input circuits is read-only. It always reflects the actual state of the circuit at the instant of reading. |
| --- | --- |
| Property name | STATE |
| Data type | Named constant |
| Value range | Enumerated: HIGH, LOW |

### 6.3.2	GPO STATE: State of output circuit

| Description | The STATE property of output circuits is read-only. It always reflects the actual state of the circuit at the instant of reading, but cannot be used for changing the output state. This property is not needed for basic operations therefore it may stay unimplemented, if the application does not require such capability. If implemented, it could be used for: - Command execution feedback - Logic condition checking - Synchronization of processes |
| --- | --- |
| Property name | STATE |
| Data type | Named constant |
| Value range | Enumerated: HIGH, LOW |

### 6.3.3	GPO PULSE: Command to output circuit

| Description | The PULSE property of output circuits is write-only. It determines the output circuit state transition to be executed, including both setting a continuous sate and a timed pulse of the required polarity. Full description of the transition consists of three parameters: - Level - Pulse duration - Inter-pulse time Level determines whether the high or the low logic state will be applied. Pulse duration determines how long the applied state should last. Inter-pulse time determines the pause to wait after the end of a pulse before the next transition is allowed to start. |
| --- | --- |
| Property name | PULSE |
| Data type | List of named and numeric constants |
| Value range | Level, mandatory: Enumerated: HIGH, LOW Pulse duration, optional: Positive decimal number in milliseconds Zero has special meaning – steady state. This is the default. Inter-pulse time, optional: Positive decimal number in milliseconds Zero has special meaning – pulses are allowed to overlap. The default is using the value of pulse duration Examples: PULSE=[HIGH,100,200] - high pulse, duration 100ms, pause 200ms PULSE=[HIGH,100] - high pulse, duration 100ms, pause 100ms PULSE=[HIGH,100,0] - high pulse, duration 100ms, no pause required PULSE=[HIGH,0] - high continuous state PULSE=HIGH - high continuous state |

## 6.4	Conventions and options

- The application cases below are included to illustrate the constructs that are possible in terms of this protocol, but it is not considered necessary to implement and use them all at a time. Selecting a subset to implement is a subject of application/script developer convention.

- There are concrete names proposed for all objects, properties and events in the examples below. The object and property names defined in the LWCP framework and the GPIO protocol sections above must be used exclusively. The alias names of properties and per-event property names should be viewed rather as a recommendation, and in any case they can be a device configuration option or a subject of application/script developer convention.

- Events may be associated with the STATE properties of GPIO input circuits. For the most common applications all input state changes would be handled as events. However, whether to treat a particular input state change as an event or not, it may be also determined by a different convention, or be a device configuration option, or be driven by the operation mode/state of the property owner.

- Acknowledgement messages must be always used if application level feedback is required. Even if a reliable point-to-point connection (TCP) is used, it can only guarantee reliable delivery of messages, but cannot provide feedback about message analysis and execution status.

- If UDP transport is used, either one of the following methods should be selected by a convention:

- Acknowledgements are used for delivery confirmation

- Messages are duplicated to ensure practically loss-less delivery

- Transaction identifiers must be always included to reliably link command messages with responses, if the latter are requested or expected. Even if TCP connection is used, race is still possible among concurrent operations inside the communicating entities therefore the sequence of response messages may be unpredictable.

## 6.5	GPIO events

- All GPIO input state changes belong to the event category, and they must generate event notification messages.

- Each real event is allowed to launch a single notification process exclusively. It is up to the software implementation to prevent duplicate notifications generated by alias names or dedicated per-event properties. Either a fixed algorithm or configuration options may determine which concrete property would be used to report each event.

Application case 1:

Input states are viewed as properties of individual circuits. No dedicated event properties are defined, and the actual state values are reported as events.

Subscription request:

SUB CH#1001.GPI#1 STATE

Notification reports the input state value:

EVENT CH#1001.GPI#1 STATE=HIGH

EVENT CH#1001.GPI#1 STATE=LOW

Application case 2:

Input states are viewed as properties of the LW channel. Generic alias names are defined for states of the individual circuits. No dedicated event properties are defined, and the actual state values are reported as events.

Alias names:

CH#1001.GPI#1 STATE		CH#1001 IN1_STATE

...

CH#1001.GPI#n STATE		CH#1001 INn_STATE

Subscription request:

SUB CH#1001 IN1_STATE

Notification reports the input state value:

EVENT CH#1001 IN1_STATE=HIGH

EVENT CH#1001 IN1_STATE=LOW

Application case 3:

Input states are viewed as properties of the LW channel. Meaningful alias names are defined that reflect the actual functions of circuits. CR Monitor profile is used as an example. No dedicated event properties are defined, and the actual state values are reported as events.

Alias names:

CH#1001.GPI#1 STATE		CH#1001 MUTE_CR_COMMAND

CH#1001.GPI#2 STATE		CH#1001 DIM_CR_COMMAND

CH#1001.GPI#3 STATE		CH#1001 EXT_PVW_ON_COMMAND

CH#1001.GPI#4 STATE		CH#1001 IN4_STATE

CH#1001.GPI#5 STATE		CH#1001 IN5_STATE

Subscription request:

SUB CH#1001 MUTE_CR_COMMAND

Notification reports the input state value:

EVENT CH#1001 MUTE_CR_COMMAND=HIGH

EVENT CH#1001 MUTE_CR_COMMAND=LOW

Application case 4:

Dedicated properties with meaningful names are defined for all events and attached to the LW channel. When an event occurs, the corresponding property takes value “TRUE”, which in turn triggers a notification.

Event properties:

CH#1001.GPI#1 STATE=HIGH		CH#1001 MUTE_CR_APPLIED

CH#1001.GPI#1 STATE=LOW		CH#1001 MUTE_CR_RELEASED

CH#1001.GPI#2 STATE=HIGH		CH#1001 DIM_CR_APPLIED

CH#1001.GPI#2 STATE=LOW		CH#1001 DIM_CR_RELEASED

CH#1001.GPI#3 STATE=HIGH		CH#1001 EXT_PVW_APPLIED

CH#1001.GPI#3 STATE=LOW		CH#1001 EXT_PVW_RELEASED

CH#1001.GPI#4 STATE=HIGH		no event associated

CH#1001.GPI#4 STATE=LOW		no event associated

CH#1001.GPI#5 STATE=HIGH		no event associated

CH#1001.GPI#5 STATE=LOW		no event associated

Subscription request to all events of channel #1001:

SUB CH#1001

Notifications refer to the dedicated per-event properties:

EVENT CH#1001 MUTE_CR_APPLIED=TRUE

EVENT CH#1001 MUTE_CR_RELEASED=TRUE

EVENT CH#1001 EXT_PVW_APPLIED=TRUE

## 6.6	GPIO commands

- A command can be sent to an output circuit by means of writing the command data to its PULSE property.

- The requested output state transition must be started immediately upon receiving the command. The only exception is the case where commands follow rapidly one after the other, so that the inter-packet pause requested by the previous command is not expired yet.

- GPIO output states follow commands and must not generate any notification messages, besides the confirmations as requested by the command message.

Application case 1:

Output commands are viewed as properties of individual circuits therefore they are addressed to circuits.

Setting continuous high logic state:

SET CH#1001.GPO#1 PULSE=HIGH

Sending a timed pulse: high logic state, duration 100ms, pause 200ms:

SET CH#1001.GPO#1 PULSE=[HIGH,100,200]

Application case 2:

Output commands are viewed as properties of a LW channel therefore they are addressed to the channel.

Alias names are defined for pulse properties of the individual circuits. Line source profile used as an example:

CH#1001.GPO#1 PULSE		CH#1001 ON_LAMP

CH#1001.GPO#2 PULSE		CH#1001 OFF_LAMP

CH#1001.GPO#3 PULSE		CH#1001 PVW_LAMP

CH#1001.GPO#4 PULSE		CH#1001 START_PULSE

CH#1001.GPO#5 PULSE		CH#1001 STOP_PULSE

Turning off the preview lamp:

SET CH#1001 PVW_LAMP=LOW

Start pulse, duration 100ms, pause 100ms:

SET CH#1001 START_PULSE=[HIGH,100]

## 6.7	GPIO configurations

### 6.7.1	Console – source/monitor, UDP multicast

- The common GPIO multicast channel 239.192.255.4 is used for all communications.

- Each GPIO port is assigned to a LW channel, and is receiving and transmitting on it. Console uses this channel to communicate with the corresponding source or monitor.

- Since all messages are multicast, there is no need for subscription – the receiver selects what it needs to be listening to.

- Since UDP transport is used, acknowledgements must be requested for delivery confirmation, or messages must be duplicated, according to a convention.

- Channel numbers must be included in the response messages to assist selective processing at m/c receivers.

- Message sequence examples.

  - Acknowledgements are used for delivery confirmation:

Source GPIO detects an input state change – TALK command from source

SOURCE:  EVENT CH#1001 TALK_COMMAND=HIGH $ACK $TRXI=27

CONSOLE: ACK CH#1001 $TRXI=27

Console turns on the TALK lamp

CONSOLE: SET CH#1001 TALK_LAMP=HIGH $ACK $TRXI=505

SOURCE:  ACK CH#1001 $TRXI=505

  - Messages are duplicated. $TRXI used to link the duplicates:

Source GPIO detects an input state change – TALK command from source

SOURCE:  EVENT CH#1001 TALK_COMMAND=HIGH $TRXI=27

SOURCE:  EVENT CH#1001 TALK_COMMAND=HIGH $TRXI=27

CONSOLE:

Console turns on the TALK lamp

CONSOLE: SET CH#1001 TALK_LAMP=HIGH $TRXI=505

CONSOLE: SET CH#1001 TALK_LAMP=HIGH $TRXI=505

SOURCE:

### 6.7.2	Console – source/monitor, TCP unicast

- TCP connections are opened between the console and each of GPIO devices.

- Each GPIO port is assigned to a LW channel. Console uses this channel number to address the port in the device.

- Messages are copied individually to each recipient therefore subscription is required.

- Since TCP transport is used, acknowledgements are optional, except for operations, which require application level feedback. Whether to request them or not, in each particular case it should be the decision of the application logic designer.

- TCP connection isolates the receiver from message flows of other recipients, and transaction identifier is sufficient for linking messages, therefore including the channel number in response messages is formally redundant. However, having the channel number may appear convenient from the implementation viewpoint, therefore using it or not should be a subject of convention.

- Message sequence example:

Console subscribes to all events of a source and the CR monitor, ACK requested

CONSOLE: SUB CH#1001 $ACK $TRXI=506

SOURCE:  ACK $TRXI=506

CONSOLE: SUB CH#2101 $ACK $TRXI=507

MONITOR: ACK $TRXI=507

Source GPIO detects an input state change – TALK command from source

SOURCE:  EVENT CH#1001 TALK_COMMAND=HIGH

CONSOLE:

Console turns on the TALK lamp

CONSOLE: SET CH#1001 TALK_LAMP=HIGH

SOURCE:

### 6.7.3	GPIO snake, TCP unicast

- TCP connection is opened between every two devices to be operating in pair.

- Each GPIO port is assigned to a LW channel. Inputs are sending and outputs are receiving event notifications on this channel.

- Messages are copied individually to each recipient therefore subscription is required.

- Since TCP transport is used, acknowledgements are optional, except for operations, which require application level feedback. Whether to request them or not, in each particular case it should be the decision of the application logic designer.

- TCP connection isolates the receiver from message flows of other recipients, and transaction identifier is sufficient for linking messages, therefore including the channel number in response messages is formally redundant. However, having the channel number may appear convenient from the implementation viewpoint, therefore using it or not is a subject of convention.

- Only the receiver’s internal logic, not this protocol, determines how to react on a received event notification. For the snake configuration to work, the GPIO device must be somehow told to be activating outputs upon input event notifications from other devices. A built-in fixed rule or a configuration option could determine how to link in pairs source ports and destination ports of different units.

- Message sequence example.

This example illustrates a complex setup with three devices:

- A is relaying events from B ports #1,2

- B is relaying events from A ports #1,2

- C is relaying events from A port #1

Unit A subscribes to all events of B, ports 1 and 2

GPIO A:  SUB CH#2001 $ACK $TRXI=11			 GPIO B

GPIO B:  ACK $TRXI=11

GPIO A:  SUB CH#2002 $ACK $TRXI=12			 GPIO B

GPIO B:  ACK $TRXI=12

Unit B subscribes to all events of A, ports 1 and 2

GPIO B:  SUB CH#1001 $ACK $TRXI=21			 GPIO A

GPIO A:  ACK $TRXI=21

GPIO B:  SUB CH#1002 $ACK $TRXI=22			 GPIO A

GPIO A:  ACK $TRXI=22

Unit C subscribes to all events of A, port 1

GPIO C:  SUB CH#1001 $ACK $TRXI=31			 GPIO A

GPIO A:  ACK $TRXI=31

GPIO A detects an input state change on port 1

GPIO A:  EVENT CH#1001 IN4_STATE=HIGH $ACK $TRXI=13	 GPIO B

GPIO A:  EVENT CH#1001 IN4_STATE=HIGH $ACK $TRXI=14	 GPIO C

GPIO B:  ACK $TRXI=13					 GPIO A

GPIO B:  (set output circuit#4 of port#1: HIGH)

GPIO C:  ACK $TRXI=14					 GPIO A

GPIO C:  (set output circuit#4 of port#1: HIGH)

## 6.8	Proposed approaches for overcoming hardware capacity limitations

- The existing GPIO hardware has limited port/circuit capacity, which may appear insufficient for certain kinds of applications.

- The GPIO application protocol is not tied to any fixed hardware configuration, and it operates with logical object entity numbers/IDs rather than physical. That allows using practically unlimited addressing capacity:

- The 32K channel number limit is inherited from the LW streaming architecture, and the GPIO protocol allows exploiting all this capacity to identify GPIO circuit collections

- Unlimited number of circuits per channel, since there is no limit for the number of ASCII digits in the protocol messages

- Several hardware ports may be assigned the same LW channel number to add more circuits to the channel. This is not fundamentally limited to a single hardware device. As long as user can ensure non-conflicting circuit numbering either manually or with assistance of some automatic discovery and configuration protocol, the linked ports may be located arbitrarily in many different devices

- The simplest user interface method for configuring circuit numbers across multiple ports could be manually assigning a base parameter to each hardware port. Then the logical number of a circuit would be calculated as the base, plus the physical number, minus 1. Example:

GPIO port #1 - assign:	Channel=1001, Base=1

ID of input circuit #1:	CH#1001.GPI#1

...

ID of input circuit #5:	CH#1001.GPI#5

ID of output circuit #1:	CH#1001.GPO#1

...

ID of output circuit #5:	CH#1001.GPO#5

GPIO port #4 - assign:	Channel=1001, Base=6

ID of input circuit #1:	CH#1001.GPI#6

...

ID of input circuit #5:	CH#1001.GPI#10

ID of output circuit #1:	CH#1001.GPO#6

...

ID of output circuit #5:	CH#1001.GPO#10

- Assigning two independent base parameters for inputs and outputs would bring slightly more sophisticated configuration capabilities. Example:

GPIO port #1 - assign:	Channel=1001, Base for inputs=3, Base for outputs=5

ID of input circuit #1:	CH#1001.GPI#3

...

ID of input circuit #5:	CH#1001.GPI#7

ID of output circuit #1:	CH#1001.GPO#5

...

ID of output circuit #5:	CH#1001.GPO#9

- If necessary, an independent number could be manually assigned per circuit. Example:

GPIO port #1 - assign:	Channel=1001

Assign number to input circuit #1:	3

Obtained ID of input circuit #1:	CH#1001.GPI#3

Assign number to input circuit #2:	44

Obtained ID of input circuit #2:	CH#1001.GPI#44

Assign number to input circuit #3:	555

Obtained ID of input circuit #3:	CH#1001.GPI#555

- Alias names can be used to associate properties immediately with the channel. There is no limitation to the addressing capacity in this case as well. Example:

GPIO port #1 - assign:	Channel=1001

Assign alias name to pulse property of output circuit #1:	ON_LAMP_01

Pulse property of output circuit #1 can be accessed as:	CH#1001 ON_LAMP_01

...

Assign alias name to pulse property of output circuit #5:	ON_LAMP_05

Pulse property of output circuit #5 can be accessed as:	CH#1001 ON_LAMP_05

...

GPIO port #5 - assign:	Channel=1001

Assign alias name to pulse property of output circuit #1:	ON_LAMP_21

Pulse property of output circuit #1 can be accessed as:	CH#1001 ON_LAMP_21

...

Assign alias name to pulse property of output circuit #5:	ON_LAMP_25

Pulse property of output circuit #5 can be accessed as:	CH#1001 ON_LAMP_25

# 7.	User Panel Application Protocol

## 7.1	Application

The User Panel Application Protocol is intended to provide interface between all kinds of user interface controls and application entities controlled by them and/or using them for output.

Eventually the protocol may have to support up to all of the following control types, and maybe even more:

- Single keys

- Rotary controls

- Joysticks

- Keypads

- LEDs

- Text/bitmap displays

- Graphical screens

Many practical applications may need just part of this variety, therefore it is acceptable to not specify and implement support for all control types and all properties immediately, but add them gradually as the requirements evolve.

## 7.2	User panel application objects

### 7.2.1	General

- Objects of the user panel application are user interface controls of various types.

- Several different object classes are defined for various types of interface controls. The class membership of the object determines its properties.

- Some properties are essentially equivalent for different object classes. In such cases common property definitions apply.

### 7.2.2	Object addressing models

- Two alternative addressing models are defined:

  - Addressing based on information flows. In terms of this model a LW channel is assigned to a certain subset of application operations as the communications media. All interface controls used by these operations are viewed as elements of the assigned LW channel. Each interface control has a unique entity identifier in terms of the channel as a super-object.

Full reference to an interface control, using the channel-based addressing model:

CH#<channel number>.<Interface control entity ID>

  - Addressing based on physical hardware. In terms of this model all interface controls are addressed relative to the physical hardware module where they are installed. Each interface control has a unique entity identifier in terms of its hardware module.

The class names for hardware modules are:

MOD_USER	– User panel

(Other classes for different module types may be defined later)

The entity identifier is a positive integer decimal number, and the object identifier for the whole hardware module is:

MOD_USER#<module number>

Full reference to an interface control, using the hardware-based addressing model:

MOD_USER#<module number>.<Interface control entity ID>

  - Preferring one or the other addressing model may depend on the hardware and software structure and configuration methods of the involved application entities. Selecting one of these models is up to a convention by application/script developers.

### 7.2.3	Control types

#### 7.2.3.1	Button

The “button” control type applies to devices consisting of the following:

- A single two-state switch for command input

- Optionally, a single indication device, mechanically or logically coupled with the switch. This indication device supports only limited text and graphical output capabilities. Another control type must be used for full-featured graphical screens. Incandescent bulbs are not distinguished from single-colored LEDs.

The class name for controls of the “button” type is:

BUT

The entity identifier is a positive integer decimal number, and the full object identifier is one of the following:

CH#<channel number>.BUT#<control number>

MOD_USER#<module number>.BUT#<control number>

The following properties are defined for controls of the “button” type:

KEY			- read-only

IND			- write-only

TEXT			- write-only

BITMAP			- write-only

ICON			- write-only

FORECOLORON		- write-only

BACKCOLORON		- write-only

FORECOLOROFF		- write-only

BACKCOLOROFF		- write-only

The protocol supports just a single picture stored in an indication device at a time. The TEXT, BITMAP, and ICON properties represent three mutually exclusive methods for specifying the picture. Writing to any one of them clears the contents of the rest two properties and replaces the previous picture with the new one.

#### 7.2.3.2	Text/bitmap display

The “display” control type applies to devices consisting of a single indication device. This indication device supports only limited text and graphical output capabilities. Another control type must be used for full-featured graphical screens. Incandescent bulbs are not distinguished from single-colored LEDs.

Formally this control type is equivalent to the indication component of the composite “button” object, it is a subset of the “button”. A dedicated control type is introduced for it exclusively to avoid confusion that may arise from using a name that obviously does not match the function of the actual hardware device.

The class name for controls of the “display” type is:

DISP

The entity identifier is a positive integer decimal number, and the full object identifier is one of the following:

CH#<channel number>.DISP#<control number>

MOD_USER#<module number>.DISP#<control number>

The following properties are defined for controls of the “display” type:

IND			- write-only

TEXT			- write-only

BITMAP			- write-only

ICON			- write-only

FORECOLORON		- write-only

BACKCOLORON		- write-only

FORECOLOROFF		- write-only

BACKCOLOROFF		- write-only

The protocol supports just a single picture stored in an indication device at a time. The TEXT, BITMAP, and ICON properties represent three mutually exclusive methods for specifying the picture. Writing to any one of them clears the contents of the rest two properties and replaces the previous picture with the new one.

#### 7.2.3.3	Other control types - TBD

- Rotary control

- Joystick

- Keypad

- LED

- Graphical screen

## 7.3	Properties

### 7.3.1	KEY: State of the key

| Description | The KEY property is read-only. It always reflects the actual state of the key at the instant of reading. Depending on the control type, this property may be associated with a simple push-button or the click function of a rotary control or joystick. |
| --- | --- |
| Property name | KEY |
| Data type | Named constant |
| Value range | Enumerated: UP, DOWN |

### 7.3.2	IND: State of the indication device

| Description | The IND property is write-only. It determines the state of the indication device as a certain ON-OFF cadence. The value of this property is the name of a predefined periodic blinking pattern, which determines the actual cadence, including steady on/off states as special cases. Once a pattern is selected, the indication device periodically switches the fore and back colors between the pairs defined for ON and OF states respectively, see the color control properties below. |
| --- | --- |
| Property name | IND |
| Data type | Named constant |
| Value range | Enumerated: ON – continuous ON state OFF – continuous OFF state FLASH_1000 – symmetric period: 1000ms ON / 1000ms OFF FLASH_500 – symmetric period: 500ms ON / 500ms OFF FLASH_250 – symmetric period: 250ms ON / 250ms OFF FLASH_125 – symmetric period: 125ms ON / 125ms OFF FLASH_375_125 – asymmetric period: 375ms ON / 125ms OFF DBL_WINK_125 – 125ms ON / 125ms OFF/ 125ms ON / 625ms OFF TRPL_WINK_125 – 125ms ON / 125ms OFF/ 125ms ON / 125ms OFF / 125ms ON / 375ms OFF DBL_WINK_125_500 – 125ms ON / 125ms OFF/ 500ms ON / 250ms OFF More enumerated values may be added to the list as long as they are supported by hardware. Notes: Although no formal restriction applies to pattern names, for clarity sake it is recommended that the names are meaningful and comply with a common style. Obviously, the pattern names above are constructed following certain logic, still trying to define a universal formal algorithm seems impractical. Therefore the primary goal, when giving new names, should be intuitiveness of each individual name taken separately, as well as in context with the other names. This is the logic behind the names defined above: - All patterns are built on the basis of a common smallest time unit of 125ms. All ON/OFF intervals are multiples of 125ms. Time interval numbers appearing in names are expressed in milliseconds. - All “FLASH” patterns have simple cycle consisting of one ON interval and one OFF interval, and the name reflects the pulse duration and on/off ratio in the cadence. - The “WINK” patterns are groups of pulses repeated once in a longer period, and the name reflects the number and duration of pulses in the period. Examples: IND=ON - continuous ON state IND=FLASH_500 - flashing 500ms – 500ms IND=DBL_WINK_125 - two pulses following rapidly, 125ms each, series repeated with a period of 1 second |

### 7.3.3	TEXT: ASCII characters to display

| Description | The TEXT property is write-only. It determines the text to be displayed on the indication device. Except line breaks, which are allowed in the text value, the protocol does not support any other layout control. Fixed font and number of character positions on the display is assumed. The characters will be displayed starting from the top left display position to the right bottom position. If the display area is organized in more than a single line, the first line will be filled to the end, after which the first character of the second line will be filled, and so continue until the last character in the command. If the number of characters in the command is less than the display area, the remaining display positions shall be cleared. Characters not fitting in the display area shall be ignored. Line break, if encountered, will skip the end of the current line and position the next character at the beginning of the next display line. |
| --- | --- |
| Property name | TEXT |
| Data type | Literal constant |
| Value range | No content or length limitations in the protocol. The indication device capacity determines the final display. Examples for a single-row, 12-char display: TEXT=”Studio Guest” - Full display TEXT=”Host Mic ” - Full display. The trailing spaces overwrite the previous content TEXT=”Host Mic” - Produces the same result as above, the missing positions are cleared. TEXT=”” - Clears all display area Examples for a display 3 rows by 6 chars: TEXT=”source HOST MIC ” - Produces display: ¦source¦ ¦ HOST ¦ ¦ MIC ¦ The same can be achieved by means of using line breaks: TEXT=”source/n HOST/n MIC” - Produces display: ¦source¦ ¦ HOST ¦ ¦ MIC ¦ TEXT=”SELECT” - Produces display: ¦SELECT¦ ¦ ¦ ¦ ¦ TEXT=” SELECT xyz” - Produces display: ¦ ¦ ¦SELECT¦ ¦ ¦ TEXT=”” - Clears the display: ¦ ¦ ¦ ¦ ¦ ¦ |

### 7.3.4	BITMAP: Bitmap pattern to display

| Description | The BITMAP property is write-only. It determines a single-color, two-state (on/off) bitmap pattern to be displayed on the indication device. The pattern is coded in an array of byte values. The physical format of the display area determines what data mapping scheme is most efficient and intuitive in each particular case, therefore no universal rule is specified by the protocol, and it is left up to the application/script developers. If the amount of data in the command is less than the display area, the missing bits should be stuffed with zeros (turned off) by the resource. Data not fitting in the display area shall be ignored. |
| --- | --- |
| Property name | BITMAP |
| Data type | Array of byte values in hex presentation. |
| Value range | No content or length limitations in the protocol. The indication device capacity determines the final display. Examples for a LED matrix of 5 rows by 7 columns: Data mapping convention: one byte per row, starting from top, low-order bits fill the row from right, the highest bit of each byte is ignored. BITMAP=[0x70,0x70,0x06] - Produces display: ¦xxx ¦ ¦xxx ¦ ¦ xx ¦ BITMAP=[0xF0,0xF0,0x86,0x55] - Produces the same display. The excess high-order bits and the whole excess byte were ignored: ¦xxx ¦ ¦xxx ¦ ¦ xx ¦ BITMAP=[0x70,0x70] - The missing bits were stuffed with zeros: ¦xxx ¦ ¦xxx ¦ ¦ ¦ BITMAP=0 - Clears the display: ¦ ¦ ¦ ¦ ¦ ¦ |

### 7.3.5	ICON: Icon to display

| Description | The ICON property is write-only. It determines a predefined graphical image to be displayed on the indication device. The data sets that define images are referenced by names. Matching of the image data with the physical display device must be ensured in the resource internally. If an undefined image name is specified in the command, it must be ignored, and the display must stay unchanged. |
| --- | --- |
| Property name | ICON |
| Data type | Named constant |
| Value range | Enumerated names, assigned to predefined images by a convention. Examples: ICON=SATELLITE ICON=LOCKED |

### 7.3.6	FORECOLORON: foreground color for ON state
BACKCOLORON: background color for ON state
FORECOLOROFF: foreground color for OFF state
BACKCOLOROFF: background color for OFF state

| Description | Philosophy: These 4 color control properties implement abstraction from the hardware design. Regardless of the indication device type, the user application should specify the required visual effect, rather than directly drive hardware elements of the indication device. This isolates the user scripts from knowledge of the specific hardware technology and physics behind forming the graphical image. The achieved end-user effect is that the user scripts appear immune against hardware variance across module versions, which may be encountered in result of gradually adding new modules manufactured in different times, as well as replacing faulty or obsolete modules. Properties: The 4 color control properties determine two color pairs – one fore and one back color per each of the 2 indication device states – turned on or off. The same properties control colors regardless of the pattern type – continuous state or blinking. Indication device switches between the two color pairs according to the cadence determined by the blinking pattern. All these properties are write-only. - The FORECOLORON property determines the color of a multi-color LED, or the color of the picture on a text/bitmap display when it is turned on. - The BACKCOLORON property determines the color of the background on a text/bitmap display when it is turned on. - The FORECOLOROFF property determines the color of a multi-color LED, or the color of the picture on a text/bitmap display when it is turned off. - The BACKCOLOROFF property determines the color of the background on a text/bitmap display when it is turned off. The universal method for color encoding is RGB, 1 data byte per color. Enumerated color names may be assigned by a convention to selected RGB values. This allows using more compact and readable commands if the number of color variants is small, as well as for most frequently needed operations. RGB values not supported by the indication device must be replaced with the closest match in the resource. This color translation rule is not defined by the protocol and is completely up to the implementation. |
| --- | --- |
| Property name | FORECOLORON BACKCOLORON FORECOLOROFF BACKCOLOROFF |
| Data type | - Array of 3 byte values in decimal or hex presentation for arbitrary RGB - Named constant for enumerated predefined colors |
| Value range | Each byte in the RGB array determines the intensity of the corresponding color in the range 0..255, where 0 is off, and 255 is max. Examples for arbitrary RGB: FORECOLORON=[255,255,255] - white full brightness FORECOLORON=[0xFF,0xFF,0xFF] - white full brightness BACKCOLORON=[128,128,128] - white half brightness FORECOLORON=[255,0,0] - red full brightness FORECOLOROFF=[0,255,0] - green full brightness FORECOLOROFF=[0,0,0] - off (black) Examples for predefined colors: FORECOLORON=WHITE is equivalent to FORECOLORON=[255,255,255] FORECOLORON=BLUE is equivalent to FORECOLORON=[0,0,255] |

## 7.4	Message sequence examples

### 7.4.1	Key events

Application case 1:

Key states are viewed as properties of individual controls, addressed by hardware module. No dedicated event properties are defined, and the actual state values are reported as events.

Subscribe to key events

C: SUB MOD_USER#12.BUT#4 KEY

S:

Key pressed and released

S: EVENT MOD_USER#12.BUT#4 KEY=DOWN

C:

S: EVENT MOD_USER#12.BUT#4 KEY=UP

C:

Application case 2:

Key states are viewed as properties of a LW channel. Meaningful alias names are defined that reflect the actual functions of keys. No dedicated event properties are defined, and the actual key state values are reported as events.

Alias names:

CH#1001.BUT#1 KEY		CH#1001 KEY_ROUTE1

CH#1001.BUT#2 KEY		CH#1001 KEY_ROUTE2

CH#1001.BUT#3 KEY		CH#1001 KEY_ROUTE3

CH#1001.BUT#4 KEY		CH#1001 KEY_TAKE

Subscribe to all key events on channel

C: SUB CH#1001

S:

Key “Select route 2” pressed and released

S: EVENT CH#1001 KEY_ROUTE2=DOWN

C:

S: EVENT CH#1001 KEY_ROUTE2=UP

C:

Key “Take selection” pressed and released

S: EVENT CH#1001 KEY_TAKE=DOWN

C:

S: EVENT CH#1001 KEY_TAKE=UP

C:

Application case 3:

Dedicated properties with meaningful names are defined for all events and attached to the LW channel. When an event occurs, the corresponding property takes value “TRUE”, which in turn triggers a notification.

Event properties:

CH#1001.BUT#1 KEY=DOWN		CH#1001 ROUTE1_SELECTED

CH#1001.BUT#1 KEY=UP		no event associated

CH#1001.BUT#2 KEY=DOWN		CH#1001 ROUTE2_SELECTED

CH#1001.BUT#2 KEY=UP		no event associated

CH#1001.BUT#3 KEY=DOWN		CH#1001 ROUTE3_SELECTED

CH#1001.BUT#3 KEY=UP		no event associated

CH#1001.BUT#4 KEY=DOWN		CH#1001 TAKE_COMMAND

CH#1001.BUT#4 KEY=UP		no event associated

Subscribe to all key events on channel

C: SUB CH#1001

S:

Key “Select route 2” pressed and released

S: EVENT CH#1001 ROUTE2_SELECTED=TRUE

C:

Key “Take selection” pressed and released

S: EVENT CH#1001 TAKE_COMAND=TRUE

C:

### 7.4.2	Commands to indication device

Turn on a plain LED, addressed to a hardware module:

C: SET MOD_USER#12.BUT#4 IND=ON

S:

Set colors and flash a multi-color LED, addressed to a hardware module:

C: SET MOD_USER#12.BUT#5 FORECOLORON=RED,FORECOLOROFF=GREEN

S:

C: SET MOD_USER#12.BUT#5 IND=FLASH_500

S:

Set colors and write text, 3 x 6 chars, to an LCD button, addressed to a LW channel:

C: SET CH#1001.BUT#6 FORECOLORON=BLACK,BACKCOLORON=WHITE,

FORECOLOROFF=BLACK,BACKCOLOROFF=BLACK

S:

C: SET CH#1001.BUT#6 TEXT=”source HOST  MIC  ”

S:

C: SET CH#1001.BUT#6 IND=ON

S:

Change the background color, replace the formerly written text on the LCD button with an icon, start flashing rapidly:

C: SET CH#1001.BUT#6 BACKCOLORON=BLUE

S:

C: SET CH#1001.BUT#6 ICON=SATELLITE

S:

C: SET CH#1001.BUT#6 IND=FLASH_125

S:

Set colors and write text, 10 chars in a single line, to a green LED alpha matrix:

C: SET MOD_USER#12.DISP#1 FORECOLORON=WHITE,BACKCOLORON=BLACK,

FORECOLOROFF=BLACK,BACKCOLOROFF=BLACK

S:

C: SET MOD_USER#12.DISP#1 TEXT=”Player 01 ”

S:

C: SET MOD_USER#12.DISP#1 IND=ON

S:

# 8.	Console Remote Control Protocol

## 8.1	Application

The Console Remote Control Protocol (CRCP) is intended to provide interface between the console application control entity and all kinds of external entities with the following purposes:

- Controlling the console operation as a reaction on certain conditions in the external entity

- Observing certain aspects of the console operation, to provide reaction in the external entity

CRCP supports basic access control using a simple login procedure.

This protocol deals with the actual operation states and the relevant controls in application components. Although some operation control attributes may have common names with the related items in configuration data tables, it must be understood that this protocol is not intended for direct viewing and editing of the stored configuration data. If such access in terms of LWCP would ever be needed, a dedicated remote configuration protocol with appropriate interface objects would be introduced for this purpose.

## 8.2	Common application control

### 8.2.1	Objects

At the root level the application control entity represents taken together all the software components that implement the console operation logic.

The class name for the application control entity is:

AppControl

As long as there is just one such entity per console, no entity identifier is needed.

Note for future developments:

If needed, individual components of the console application would be addressable as nested objects, lower by one structure level. For example:

AppControl.UIL	- User interface layer

AppControl.SHC	- Show profile control component

### 8.2.2	Properties

#### 8.2.2.1	Overview

The following properties are defined:

ShowProfID		- Show profile identifier

ShowProfName		- Show profile name

ShowProfStat		- Show profile loading status

GpioList		- GPIO resource list

fader_list		- List of the actually installed fader numbers	RQ 4610

Note, 18 Sep 2009:	The fader_list property is an interim solution. It is planned to be replaced by an XML structure for significantly extended and open functionality.

#### 8.2.2.2	ShowProfID: Show profile identifier

| Description | Object: AppControl The ShowProfID property is read-and-write. Writing to this property determines the show profile to be loaded. Reading it returns the currently selected show profile identifier. Whenever the current show profile changes for any reason, the console must generate an event notification, including the new ID and name, and loading status. |
| --- | --- |
| Property name | ShowProfID |
| Data type | Numeric, hex or decimal, positive integer |
| Value range | All 32-bit values are allowed |

#### 8.2.2.3	ShowProfName: Show profile name

| Description | Object: AppControl The ShowProfName property is read-only. Reading it returns the name of the currently selected show profile as defined in show profile data. |
| --- | --- |
| Property name | ShowProfName |
| Data type | Literal constant |
| Value range | No content or length limitations are defined by this protocol. The actual format and content are determined by show profile data. |

#### 8.2.2.4	ShowProfStat: Show profile loading status

| Description | Object: AppControl The ShowProfStat property is read-only. Reading it returns the actual status of the currently selected show profile. |
| --- | --- |
| Property name | ShowProfStat |
| Data type | Named enumerated constant |
| Value range | LOADING, READY |

#### 8.2.2.5	GpioList: GPIO resource list

| Description | Object: AppControl The GpioList property is read-only. Reading it returns the list of GPIO circuits currently in use by the console application. Each list entry consists of a circuit ID and function description. This property does not deal with stored configuration data. |
| --- | --- |
| Property name | GpioList |
| Data type | Encapsulated data |
| Value range | Document with the following structure: <List> <Input> <Channel>(LW channel number) </Channel> <Circuit>(Circuit number)</Circuit> <Function>(Textual description)</Function> </Input> ... <Output> <Channel>(LW channel number) </Channel> <Circuit>(Circuit number)</Circuit> <Function>(Textual description)</Function> </Output> ... </List> Empty list: <List></List> |

#### 8.2.2.6	fader_list: List of the actually installed fader numbers

RQ 4610

| Description | Object: AppControl The fader_list property is read-only. Reading it returns the list of actually installed fader numbers. |
| --- | --- |
| Property name | fader_list |
| Data type | List |
| Value range | The list contains positive integers, corresponding to the fader channel numbers. |

### 8.2.3	Commands and events

#### 8.2.3.1	Legend

C		Client		Represents the external entity communicating with the console application

S		Server		Represents the console application as a subject of CRCP

Console				Represents the internal algorithms of the console application

#### 8.2.3.2	Trigger a profile change

The command to change profile can be given only using the profile ID. The name property is informative only.

C: SET AppControl ShowProfID=<new profile ID>

S: EVENT AppControl ShowProfID=<ID>,ShowProfName=<name>,ShowProfStat=LOADING

Note:	An explicit ACK or INDI may be requested in the SET command, but it seems redundant here, since the event notification should follow automatically.

#### 8.2.3.3	Determine what the current profile and its status is

Read all in a single command:

C: GET AppControl ShowProfID,ShowProfName,ShowProfStat

S: INDI AppControl ShowProfID=<ID>,ShowProfName=<name>,ShowProfStat=READY

Read a single property in a command:

C: GET AppControl ShowProfName

S: INDI AppControl ShowProfName=<name>

#### 8.2.3.4	Observe profile changes

Any change of any one or several attributes from the group {ShowProfID, ShowProfName, ShowProfStat} will trigger an event report. All three properties will be included in all event reports.

To receive these profile change notifications, it is required and sufficient that the client were subscribed to the profile loading status property.

Console operator: change profile via console interface

Console: start loading the new profile

S: EVENT AppControl ShowProfID=<ID>,ShowProfName=<name>,ShowProfStat=LOADING

Console: loading the new profile completed

S: EVENT AppControl ShowProfID=<ID>,ShowProfName=<name>,ShowProfStat=READY

#### 8.2.3.5	Obtain a list of GPIO resources

C: GET AppControl GpioList

S: INDI AppControl GpioList=%BeginEncap%(list data goes here)%EndEncap%

Returning empty list:

S: INDI AppControl GpioList=%BeginEncap%<List></List>%EndEncap%

#### 8.2.3.6	Obtain a list of actually installed physical fader numbers

RQ 4610

C: GET AppControl fader_list

S: INDI AppControl fader_list=[1,2,3,4,11,12]

Returning empty list:

S: INDI AppControl fader_list=[]

### 8.2.4	Message sequence examples

These operations use a TCP point-point connection. Guaranteed delivery is assumed.

Client logs in to the console application control. Only basic login is presently defined, no access levels.

C: LOGIN AppControl USER=MIKE,PWD=AXIA $ACK $TRXI=555

S: ACK $TRXI=555 $STATUS=OK

Client subscribes to profile change events

C: SUB AppControl ShowProfStat

Client inquires the current profile identity and status

C: GET AppControl ShowProfID,ShowProfName,ShowProfStat

S: INDI AppControl ShowProfID=1,ShowProfName=”Default”,ShowProfStat=READY

Console operator selects a profile via local interface, console starts loading, and a notification is reported.

Console operator: change profile via console interface

Console: start loading the new profile

S: EVENT AppControl ShowProfID=2,ShowProfName=”Mike’s Morning”,ShowProfStat=LOADING

C: <Do whatever is required as the reaction>

Profile completes loading

Console: loading the new profile completed

S: EVENT AppControl ShowProfID=2,ShowProfName=”Mike’s Morning”,ShowProfStat=READY

C: <Do whatever is required as the reaction>

Client obtains the profile list using the resource discovery protocol …

C: GET ConfigData ShowProfList

S: INDI ConfigData ShowProfList=%BeginEncap%

<List>

<ShowProfile>

<ID>1</ID>

<Name>Default</Name>

</ShowProfile>

<ShowProfile>

<ID>2</ID>

<Name>Mike’s Morning</Name>

</ShowProfile>

<ShowProfile>

<ID>3</ID>

<Name>Night Music</Name>

</ShowProfile>

</List>

%EndEncap%

… and triggers changing the active profile on console

C: SET AppControl ShowProfID=3

Console: start loading the new profile

S: EVENT AppControl ShowProfID=3,ShowProfName=”Night Music”,ShowProfStat=LOADING

Console: loading the new profile completed

S: EVENT AppControl ShowProfID=3,ShowProfName=”Night Music”,ShowProfStat=READY

Client obtains from application control the list of currently used GPIO resources …

C: GET AppControl GpioList

S: INDI AppControl GpioList=%BeginEncap%

<List>

<Input>

<Channel>1001</Channel>

<Circuit>1</Circuit>

<Function>CR monitor: MUTE command</Function>

</Input>

<Output>

<Channel>1001</Channel>

<Circuit>1</Circuit>

<Function>CR monitor: ON AIR lamp</Function>

</Output>

<Output>

<Channel>1251</Channel>

<Circuit>4</Circuit>

<Function>Source: Start pulse</Function>

</Output>

</List>

%EndEncap%

… and sends a 100ms start pulse to the source on channel #1251

C: SET CH#1251.GPO#4 PULSE=[HIGH,100]

## 8.3	Console operations

### 8.3.1	Application

v.12	This protocol module is intended to be used for remote control over console fader channels, monitors, aux sends and returns.

In a typical setup the PathFinder application would act in the role of the controller. In this case a point-to-point control connection is to be established via TCP into port 4010 (decimal) on Element. The PathFinder must be aware of the controlled Element’s IP unicast address.

Application message delivery is assumed to be reliable by means of TCP, and not requiring any additional methods to be implemented in the application layer.

### 8.3.2	Objects

#### 8.3.2.1	Source ID – Livewire channel

This addressing method directs the control to the fader channel, where the specified source is currently taken, instead of a sending the control to a physical fader channel number. Since the source ID (LW channel number) is used for addressing, the control automatically follows the source to any physical fader.

The class name for LW channel is:

LwCH

The entity identifier is a LW channel number, and thus a full Object ID for a LW channel is:

LwCH #<channel number>

Example:

LwCH #1271

#### 8.3.2.2	Physical fader channel

This addressing method directs the control to a concrete fader channel number regardless of the source ID, or no source selected on it.

The class name for fader channel is:

fader

The entity identifier is the number assigned to a fader, and thus a full Object ID for a fader channel is:

fader#<fader number>

Example:

fader#3

#### 8.3.2.3	Aux return

RQ 3777

This object represents an aux return channel. Presently no substructures are defined for the aux return object.

The class name for aux return is:

aux_ret

A full Object ID for an aux return is:

aux_ret#<aux return channel number>

Example:

aux_ret#1

#### 8.3.2.4	Monitor 1

RQ 3777

This object represents the Monitor 1 circuit of the monitoring part of the console.

The class name for Monitor 1 is:

monit1

There is just one such entity per console, therefore entity identifier is not needed.

There are two addressable substructures defined in the Monitor 1 circuit, which represent the external source inputs 1 and 2. Their class names are:

ext1

ext2

There is just one entity of ext1 and one of ext2, therefore entity identifiers are not needed.

Full Object IDs for the ext source  inputs are:

monit1.ext1

monit1.ext2

#### 8.3.2.5	Monitor 2

RQ 3777

This object represents the Monitor 2 circuit of the monitoring part of the console.

The class name for Monitor 2 is:

monit2

There is just one such entity per console, therefore entity identifier is not needed.

There is one addressable substructure defined in the Monitor 2 circuit, which represents the external source input. Its class name is:

ext1

There is just one entity of ext, therefore entity identifier is not needed.

Full Object ID for the ext source  input is:

monit2.ext1

#### 8.3.2.6	Preview

RQ 3777

This object represents the Preview circuit of the monitoring part of the console.

The class name for Preview is:

prev

There is just one such entity per console, therefore entity identifier is not needed.

There is one addressable substructure defined in the Preview circuit, which represents the external source input. Its class name is:

ext1

There is just one entity of ext, therefore entity identifier is not needed.

Full Object ID for the ext source  input is:

prev.ext1

### 8.3.3	Properties

#### 8.3.3.1	Overview

The present set of properties covers only the immediately requested remote control capabilities. More properties may be added at any time to include support for other console operations.

Fader and aux return input channel control logic states. These properties can be used to remotely read or set the required state, as well as their changes can be reported as events to other applications:

ON_State		- ON/OFF state 		RQ 4738

Mute_State		- Fader channel input muting state

Asg_PGM1		- PGM1 assignment state 	RQ 4738

Asg_PGM2		- PGM2 assignment state 	RQ 4738

Asg_PGM3		- PGM3 assignment state 	RQ 4738

Asg_PGM4		- PGM4 assignment state 	RQ 4738

Input channel fader gain. This property can be used to remotely read or set the fader gain in the fader or aux return input channel. Although it is allowed too, the fader gain changes are unlikely to be reported as events to other applications:

Fader_Gain		- Fader gain	RQ 4738

Console user interface push-button states. The following properties reflect user actions. Their value changes can be reported as events to other applications:

ON_But			- State of the fader channel “ON” button

OFF_But		- State of the fader channel “OFF” button

sp_ext1_but		- State of the monitor speaker source selector “ext1” button	RQ 4736

sp_ext2_but		- State of the monitor speaker source selector “ext2” button	RQ 4736

RQ 3692   RQ 3777

Input source. These properties can be used to remotely select a source, as well as to learn what source is selected and its loading status, either by querying the console application control data, or via event reports.

src_list		- List of source profiles, filtered for the individual fader channel

src_id			- Source profile identifier

src_name		- Source profile name

src_stat		- Source profile loading status

src_lwch		- LW channel of the associated physical source

#### 8.3.3.2	ON/OFF state

| Description | Objects: LwCH, fader, aux_ret RQ 4738 This property is read and write. It allows reading and setting the fader channel or aux return On/Off state. Every state change is followed by an event notification. |
| --- | --- |
| Property name | ON_State |
| Data type | Named constant |
| Value range | Enumerated: ON,OFF |

#### 8.3.3.3	Fader channel input muting state

| Description | Objects: LwCH, fader This property is read and write. It allows reading and setting the fader channel input muting state. Every state change is followed by an event notification. |
| --- | --- |
| Property name | Mute_State |
| Data type | Named constant |
| Value range | Enumerated: NORMAL,MUTED |

#### 8.3.3.4	Fader channel mix assignment states

| Description | Objects: LwCH, fader, aux_ret RQ 4738 These properties are read and write. They allow reading and setting the fader channel or aux return mix assignment states. Every state change is followed by an event notification. Asg_PGM4 is representing a ganged switch, controlling three mix assignments synchronously – PGM4, RECORD, and PHONE.. |
| --- | --- |
| Property name | Asg_PGM1, Asg_PGM2, Asg_PGM3, Asg_PGM4 |
| Data type | Named constant |
| Value range | Enumerated: ON,OFF |

#### 8.3.3.5	Fader gain

| Description | Objects: LwCH, fader, aux_ret RQ 4738 This property is read and write. It allows reading and setting the fader gain in a fader input or aux return input channel. Event notifications for gain value changes are not required. Note: The aux return fader gain is also called “aux return master gain” in other documentation. |
| --- | --- |
| Property name | Fader_Gain |
| Data type | Decimal fixed point |
| Value range | - Represents the gain value in dB - The protocol allows arbitrary values, limited only by the data type - The surface application will trim the values received with SET commands to the following meaningful range: –80 .. +10dB, OFF (hex value 0x8000) - All values outside this numeric range will be interpreted by the application either as the maximal gain +10, or OFF, respectively - The gain resolution in surface application is 0.1dB. All higher precision values will be rounded to the closest value of the 0.1dB grid |

#### 8.3.3.6	Command button state properties

| Description | These properties are read-only. They always reflect the actual states of the corresponding buttons at the instant of reading. Every state change is followed by an event notification, which may also includes other relevant data, for example, the fader channel current On/Off state value. |
| --- | --- |
| Property name | Objects: LwCH, fader ON_But - Fader channel “ON” button OFF_But - Fader channel “OFF” button Objects: monit1 RQ 4736 sp_ext1_but - Monitor speaker source selector “ext1” button sp_ext2_but - Monitor speaker source selector “ext2” button |
| Data type | Named constant |
| Value range | Enumerated: UP, DOWN |

#### 8.3.3.7	Source profile list

RQ 3692   RQ 3777

| Description | Objects: fader aux_ret monit1.ext1 monit1.ext2 monit2.ext1 prev.ext1 This is a filtered subset of all source profiles stored in the console configuration data – the list of source profiles that are available to the corresponding console input. It is attached to the object representing the corresponding console input and is read-only. Each list entry consists of a source profile ID, source profile name, and the Livewire channel number of the associated physical source. |
| --- | --- |
| Property name | src_list |
| Data type | Encapsulated data |
| Value range | XML document with the following structure: <list> <src> <id>(profile id)</id> <name>(profile name)</name> <lwch>(livewire channel number)</lwch> </src> ... <list> Empty list: <list></list> |

#### 8.3.3.8	Source profile identifier

RQ 3692   RQ 3777

| Description | Objects: fader aux_ret monit1.ext1 monit1.ext2 monit2.ext1 prev.ext1 The src_id property is read-and-write. Writing to this property initiates an attempt to load the corresponding source profile. Reading it returns the currently selected source profile identifier. This item is included in source change event notifications. |
| --- | --- |
| Property name | src_id |
| Data type | Numeric |
| Value range | Positive integers identify profiles. Value –1 denotes none. |

#### 8.3.3.9	Source profile name

RQ 3692   RQ 3777

| Description | Objects: fader aux_ret monit1.ext1 monit1.ext2 monit2.ext1 prev.ext1 The src_name property is read-only. Reading it returns the name of the currently selected source profile. This item is included in source change event notifications. |
| --- | --- |
| Property name | src_name |
| Data type | Literal |
| Value range | Allowed printable characters, codes 32 to 126 (decimal). No other content or length limitations are defined by this protocol. The actual format and content are determined by source profile data. |

#### 8.3.3.10	Source profile loading status

RQ 3692   RQ 3777

| Description | Objects: fader aux_ret monit1.ext1 monit1.ext2 monit2.ext1 prev.ext1 The src_stat property is read-only. Reading it returns the loading status of the currently selected source profile. This item is included in source change event notifications. |
| --- | --- |
| Property name | src_stat |
| Data type | Numeric |
| Value range | Enumerated. The source status encoding is preserved as used in Element application. See appendix B in this document. |

#### 8.3.3.11	Source Livewire channel

RQ 3692   RQ 3777

| Description | Objects: fader aux_ret monit1.ext1 monit1.ext2 monit2.ext1 prev.ext1 The src_lwch property is read-only. Reading it returns the LW channel number of the physical source associated with the currently selected source profile. This item is included in source change event notifications. |
| --- | --- |
| Property name | src_lwch |
| Data type | Numeric |
| Value range | Positive integers identify LW channels, within limits of the LW capacity. Value –1 denotes none. |

### 8.3.4	Commands and events

#### 8.3.4.1	Legend

C	Client		Represents the external entity communicating with the resource

S	Server		Represents the resource as a subject of CRCP

R	Resource	Represents the internal algorithms of the resource

#### 8.3.4.2	Subscribe to fader or aux return input channel control state changes

To receive asynchronous notifications about the fader channel control state changes, the remote application must subscribe to the corresponding object. Depending on the selected addressing method, the subscription request must be issued for:

- LW channel (source ID)

C: SUB LwCH#1271 $ACK $TRXI=555

S: ACK $TRXI=555

- physical fader number

C: SUB fader#3 $ACK $TRXI=555

S: ACK $TRXI=555

- aux return		RQ 4738

C: SUB aux_ret#1 $ACK $TRXI=555

S: ACK $TRXI=555

#### 8.3.4.3	Set fader channel controls,
addressing by source LW channel number

The remote application will use SET commands to control the fader channel or aux return properties. Since this is supposed to be operated over TCP unicast, reliable delivery is assumed, and no acknowledgement is being requested.

Example 1:

Set fader channel mix assignments and the On/Off state. Remote application has not requested event notifications for control state changes:

C: SET LwCH#1271 Asg_PGM1=ON,Asg_PGM2=ON,Asg_PGM3=OFF,Asg_PGM4=OFF

S:

C: SET LwCH#1271 ON_State=ON

S:

Example 2:

Set fader channel input muting state. Remote application has requested event notifications for control state changes:

C: SET LwCH#1271 Mute_State=MUTED

S: EVENT LwCH#1271 Mute_State=MUTED

Example 3:

Set fader gain value. Fader gain values do not trigger event notifications:

C: SET LwCH#1271 Fader_Gain=-6.0

S:

Example 4:

Failed attempt to set undefined mix assignment state, transaction ID was not included:

C: SET LwCH#1271 Asg_PGM1=HIGH

S: ACK $STATUS=INVALID_VALUE

#### 8.3.4.4	Read fader channel controls,
addressing by source LW channel number

The remote application can read fader channel properties at any time using GET commands. Subscription is not required to receive value indications in response to GET commands.

Example 1:

Read fader gain:

C: GET LwCH#1271 Fader_Gain

S: INDI LwCH#1271 Fader_Gain=-6.0

Example 2:

Read fader channel PGM1 assignment and the On/Of state:

C: GET LwCH#1271 Asg_PGM1,ON_State

S: INDI LwCH#1271 Asg_PGM1=ON,ON_State=ON

#### 8.3.4.5	Notify remote application about fader channel control state changes,
addressing by source LW channel number

The resource may change fader channel control states locally. In such cases, the resource will notify the remote application about the changes by means of EVENT messages. To receive notifications, it is required and sufficient that the client were subscribed to the properties of the corresponding source (LW channel) object. Only the actually changed properties will be included in the notification.

Example 1:

Operator presses the PGM1 assignment button on a fader channel. Remote application has requested event notifications for control state changes:

User: Assign a fader to PGM1 using the control surface button

R: change the PGM1 assignment switch state

S: EVENT LwCH#1271 Asg_PGM1=ON

Example 2:

Set the fader channel input muting state remotely. The remote application has requested event notifications for control state changes, therefore an event notification is sent in response:

C: SET LwCH#1271 Mute_State=MUTED

S: EVENT LwCH#1271 Mute_State=MUTED

Example 3:

Operator presses the ON button on a fader channel, which is currently OFF. Remote application has requested event notifications for control state changes. The first event notification follows pressing the button, and it reports both the button state change and the current fader channel state. The second notification follows the requested fader channel state change. Finally the third notification follows releasing the button:

User: Press the ON button on the control surface

S: EVENT LwCH#1271 ON_But=Down,ON_State=Off

R: change the fader channel state from OFF to ON

S: EVENT LwCH#1271 ON_State=On

User: Release the ON button on the control surface

S: EVENT LwCH#1271 ON_But=Up,ON_State=On

#### 8.3.4.6	Set, get, observe fader and aux return input channel controls,
addressing by physical input channel

The handling schemes do not differ from the previous examples, excepting that another addressing method is used:

C: SET fader#3 ON_State=ON

S: EVENT fader#3 ON_State=ON

C: GET fader#3 Asg_PGM1,ON_State

S: INDI fader#3 Asg_PGM1=ON,ON_State=ON

RQ 4738

C: GET aux_ret#1 Asg_PGM1,ON_State

S: INDI aux_ret#1 Asg_PGM1=ON,ON_State=ON

C: SET aux_ret#2 Fader_Gain=-6.0

S: INDI aux_ret#2 Fader_Gain=-6.0

#### 8.3.4.7	Fader source handling

RQ 3692

// Getting source profile list for an individual fader:

C: get fader#3 src_list
S: indi fader#3 src_list=%beginencap%
<list>
    <src>
        <id>1</id>
        <name>Goog_115</name>
        <lwch>115</lwch>
    </src>
    <src>
        <id>2</id>
        <name>Goog_116</name>
        <lwch>116</lwch>
    </src>
</list>
%endencap%

// Checking the actually loaded source profile. A source is loaded on fader 3, in full control mode:

C: get fader#3 src_id, src_name, src_lwch, src_stat

S: indi fader#3 src_id=1, src_name="Goog_115", src_lwch=115, src_stat=22

// Changing the source. Source loading/unloading is followed by an event notification, regardless of what triggered the change. The new source profile is listen-only:

C: set fader#3 src_id=2

R: Load the requested source

S: event fader#3 src_id=2, src_name="Goog_116", src_lwch=116, src_stat=21

// Unloading the source:

C: set fader#3 src_id=-1

R: Unload the source

S: event fader#3 src_id=-1

// Source change notifications. The operations were initiated not in terms of this LWCP session – by the console operator, or other LWCP session:

Operator/other LWCP: Request a source change on fader#3

R: Load the requested source on fader#3

S: event fader#3 src_id=2, src_name="Goog_116", src_lwch=116, src_stat=21

Operator/other LWCP: Request unloading the source from fader#4

R: Unload source from fader#4

S: event fader#4 src_id=-1

#### 8.3.4.8	Aux return and monitor input source handling

RQ 3777

The same general handling scheme is applied here that used with fader sources:

get src_list    set src_id   event src_id

Object names used for addressing are to be substituted accordingly.

Examples:

C: get aux_ret#1 src_list
S: indi aux_ret#1 src_list=%beginencap% ... %endencap%

C: get monit1.ext1 src_id, src_name, src_lwch, src_stat

S: indi monit1.ext1 src_id=1, src_name="Goog_115", src_lwch=115, src_stat=22

C: set prev.ext1 src_id=2

S: event prev.ext1 src_id=2, src_name="Goog_116", src_lwch=116, src_stat=21

#### 8.3.4.9	Notify remote application about monitor source selector button state changes

RQ 4736

S: event monit1 sp_ext1_but=DOWN

S: event monit1 sp_ext1_but=UP

S: event monit1 sp_ext2_but=DOWN

S: event monit1 sp_ext2_but=UP

## 8.4	Fader channel – accessory module operations

### 8.4.1	Application

This protocol module is intended to be used for fader channel accessory module operations.

v.09  UDP multicast port 4011 is used to deliver the button and rotary knob events from the accessory modules to the host controller, and UDP multicast port 4012 is used to deliver commands from the host controller to the modules.

Application message delivery is assumed to be reliable by means of sending every UDP message twice. Using the message identifier $MSGI may be required to recognize the duplicates.

### 8.4.2	Objects

#### 8.4.2.1	Source ID – Livewire channel

This type of accessory module is always associated with a source, and not with any particular console and a fader number on it. The addressing method that relies on the source ID, in combination with the multicast distribution, ensure that the control automatically follows the source to any physical fader on any console.

A Livewire channel represents the source as an object using a certain isolated communications media. This Livewire channel takes the role of the addressable object in the LWCP protocol messages and thus ensures communication with the whole collection of items belonging to the source.

The class name for LW channel is:

LwCH

The entity identifier is a LW channel number, and thus a full Object ID for a LW channel is:

LwCH#<channel number>

Example:

LwCH#1271

### 8.4.3	Properties

#### 8.4.3.1	Overview

The present set of properties ensures the operations defined for the remote mic control and headphones selector accessory modules.

The following properties reflect user actions. Their value changes are handled as events, originating in the user interface and propagating to the application core:

ON_But			- State of the “ON” button

OFF_But		- State of the “OFF” button

TALK_But		- State of the “TALKBACK” button

MUTE_But		- State of the “MUTE” button

HP_Preset1_But	- State of the headphones “PRESET1” button

HP_Preset2_But	- State of the headphones “PRESET2” button

HP_Sel_But		- State of the headphones rotary selector button

HP_Sel_Rot		- Amount of rotation of the headphones rotary selector

The following properties serve for user interface indication devices control. Their values propagate from the application core to the user interface.

ON_Lamp		- State of the “ON” lamp

OFF_Lamp		- State of the “OFF” lamp

TALK_Lamp		- State of the “TALK” lamp

MUTE_Lamp		- State of the “MUTE” lamp

HP_Preset1_Lamp	- State of the headphones “PRESET1” lamp

HP_Preset2_Lamp	- State of the headphones “PRESET2” lamp

HP_Disp_Text		- Text for the headphones source name display

#### 8.4.3.2	Command button state properties

| Description | Object: LwCH These properties are read-only. They always reflect the actual states of the corresponding buttons at the instant of reading. With respect to the state property, there is no difference between the simple push-buttons and the click function of the rotary selector. |
| --- | --- |
| Property name | ON_But - State of the “ON” button OFF_But - State of the “OFF” button TALK_But - State of the “TALKBACK” button MUTE_But - State of the “MUTE” button HP_Preset1_But - State of the headphones “PRESET1” button HP_Preset2_But - State of the headphones “PRESET2” button HP_Sel_But - State of the headphones rotary selector button |
| Data type | Named constant |
| Value range | Enumerated: UP, DOWN |

#### 8.4.3.3	Command button lamp properties

| Description | Object: LwCH These properties are write-only. They determine the states of the lamps beneath the corresponding buttons. |
| --- | --- |
| Property name | ON_Lamp - State of the “ON” lamp OFF_Lamp - State of the “OFF” lamp TALK_Lamp - State of the “TALK” lamp MUTE_Lamp - State of the “MUTE” lamp HP_Preset1_Lamp - State of the headphones “PRESET1” lamp HP_Preset2_Lamp - State of the headphones “PRESET2” lamp |
| Data type | Named constant |
| Value range | Enumerated: ON – continuous ON state OFF – continuous OFF state TBD: Do we need cadences here? |

#### 8.4.3.4	Headphones selector rotation amount

| Description | Object: LwCH This property is read-only. It always contains the rotation amount relative to the last reading, expressed in rotation units (clicks). A positive value represents rotation clockwise, a negative value – counterclockwise. |
| --- | --- |
| Property name | HP_Sel_Rot |
| Data type | Decimal integer |
| Value range | Minimally required: +1, -1 represent rotation by one unit Other values may be used to represent rotation by multiple units in a single reading. |

#### 8.4.3.5	Headphones source name display text

| Description | Object: LwCH This property is write-only. It determines the text to be displayed on the indication device. Fixed font and fixed number of character positions, arranged in a single line, are assumed, and no layout control is supported. Capacity of the indication device will determine the final display. If the number of characters in the command is less than the display area, the remaining display positions shall be cleared. Characters not fitting in the display area shall be ignored (text trimmed). |
| --- | --- |
| Property name | HP_Disp_Text - Text for the headphones source name display |
| Data type | Literal constant |
| Value range | No content or length limitations in the protocol. Examples for a single-row, 10-char display: HP_Disp_Text=”Stud Guest” - Full display HP_Disp_Text=”Studio Guest” - Display “Studio Gue”, the excess characters were trimmed HP_Disp_Text=”Host Mic” - Partly filled display, the previous content was cleared HP_Disp_Text=”” - Clears all display area |

### 8.4.4	Commands and events

#### 8.4.4.1	Legend

C		Client		Represents the external entity communicating with the console application

S		Server		Represents the console application as a subject of CRCP

Console				Represents the internal algorithms of the console application

#### 8.4.4.2	User command from accessory module

Pressing or releasing any of buttons on a module, or rotating the selector will generate an event report to the application core. To avoid needlessly complicating the state logic in the communicating entities, explicit ACK to the EVENT command is not being requested. It seems redundant here, since immediate visual feedback (lamp) will be provided to the user.

Example 1: User presses the “ON” button on the module:

C: EVENT LwCH#1001 ON_But=Down

Console: (reaction to the event)

Example 2: User releases the “ON” button on the module:

C: EVENT LwCH#1001 ON_But=Up

Console: (reaction to the event)

Example 3: User rotates the selector by two clicks counterclockwise:

C: EVENT LwCH#1001 HP_Sel_Rot=-2

Console: (reaction to the event)

#### 8.4.4.3	Indication to accessory module

Application core will use SET commands to control displays on the module. To avoid needlessly complicating the state logic in the communicating entities, explicit ACK to the SET command is not being requested. It seems redundant here, since writing the content to an indication device is a trivial operation with no dependencies on other factors. It may fail only in presence of hardware or software defects.

Example 1: Turning on the “TALK” lamp on the module:

Console: (processing an operation)

S: SET LwCH#1001 TALK_Lamp=ON

C:

Example 2: Displaying a source name on the module:

Console: (processing an operation)

S: SET LwCH#1001 HP_Disp_Text=”Stud Guest”

C:

### 8.4.5	Message sequence examples

These operations employ UDP multicast. Delivery cannot be guaranteed for every single message, but it is assumed being sufficiently reliable if a pair of duplicates is sent instead of a single message. The duplicates are marked by means of identical $MSGI values. The second message of each pair is ignored, and the reaction to the pair is as if a single message was received.

Accessory module turns the channel on

C: EVENT LwCH#1001 ON_But=Down $MSGI=222

C: EVENT LwCH#1001 ON_But=Down $MSGI=222

Console: Turn the channel ON

S: SET LwCH#1001 OFF_Lamp=OFF $MSGI=444

S: SET LwCH#1001 OFF_Lamp=OFF $MSGI=444

Module: Turn off the lamp

S: SET LwCH#1001 ON_Lamp=ON $MSGI=445

S: SET LwCH#1001 ON_Lamp=ON $MSGI=445

Module: Turn on the lamp

C: EVENT LwCH#1001 ON_But=Up $MSGI=223

C: EVENT LwCH#1001 ON_But=Up $MSGI=223

Console: (no reaction)

Accessory module selects a preset source

C: EVENT LwCH#1001 HP_Preset2_But=Down $MSGI=224

C: EVENT LwCH#1001 HP_Preset2_But=Down $MSGI=224

Console: Start counting time for preset change delay

C: EVENT LwCH#1001 HP_Preset2_But=Up $MSGI=225

C: EVENT LwCH#1001 HP_Preset2_But=Up $MSGI=225

Console: Change selection to preset #2

S: SET LwCH#1001 HP_Disp_Text=”Stud Guest” $MSGI=446

S: SET LwCH#1001 HP_Disp_Text=”Stud Guest” $MSGI=446

Module: Clear the previous content and display the just received text

S: SET LwCH#1001 HP_Preset1_Lamp=OFF $MSGI=447

S: SET LwCH#1001 HP_Preset1_Lamp=OFF $MSGI=447

Module: Turn off the lamp

S: SET LwCH#1001 HP_Preset2_Lamp=ON $MSGI=448

S: SET LwCH#1001 HP_Preset2_Lamp=ON $MSGI=448

Module: Turn on the lamp

Accessory module selects a source from the common list

C: EVENT LwCH#1001 HP_Sel_Rot=1 $MSGI=226

C: EVENT LwCH#1001 HP_Sel_Rot=1 $MSGI=226

Console: Go to the next list item, read the source name

S: SET LwCH#1001 HP_Disp_Text=”Codec-1” $MSGI=449

S: SET LwCH#1001 HP_Disp_Text=”Codec-1” $MSGI=449

Module: Clear the previous content and display the just received text

C: EVENT LwCH#1001 HP_Sel_Rot=1 $MSGI=227

C: EVENT LwCH#1001 HP_Sel_Rot=1 $MSGI=227

Console: Go to the next list item, read the source name

S: SET LwCH#1001 HP_Disp_Text=”Codec-2” $MSGI=450

S: SET LwCH#1001 HP_Disp_Text=”Codec-2” $MSGI=450

Module: Clear the previous content and display the just received text

C: EVENT LwCH#1001 HP_Sel_But=Down $MSGI=228

C: EVENT LwCH#1001 HP_Sel_But=Down $MSGI=228

Console: Change selection to ”Codec-2”, switch audio

C: EVENT LwCH#1001 HP_Sel_But=Up $MSGI=229

C: EVENT LwCH#1001 HP_Sel_But=Up $MSGI=229

Console: (no reaction)

## 8.5	V-Mixer

### 8.5.1	Objects

While the V-mixer resource is physically implemented in engine, and is intended to be controlled directly, an alternative indirect control path via Element is supported too. In terms of this protocol, all V-Mixer object and property definitions are identical in the both cases.

A single root object represents the V-Mixer resource. It is a structured item, and has three structure levels:

- The V-mixer as a whole

- Submix groups

- Individual inputs to submix groups

V-Mixer object

The class name for V-Mixer object is:		VMIX

As long as there is just one V-Mixer entity per controlled device, no entity identifier is needed.

Submix group object

The class name for a submix group is:		SUB

The entity identifier is an index number, and a full Object ID for a submix group is:

SUB#<number>

Individual input object

The class name for an individual input is:	IN

The entity identifier is an index number, and a full Object ID for an input to a submix group is:

IN#<number>

Examples of the structured Object ID:

V-Mixer as a whole:				VMIX

Submix group:				VMIX.SUB#1

Input:					VMIX.SUB#1.IN#5

### 8.5.2	Properties

#### 8.5.2.1	Overview

This set of properties covers the presently defined interactive operations offered on the Element V-Mixer control web page and Engine V-Mixer control web page, excepting the external input stream connections (address, name), which are controlled via LWRP.

Submix group properties

Master_Gain		- Master gain of a submix group

Individual input properties

input_uri		- The uniform resource identifier of the input source	RQ 4519

Gain			- Gain of the fader on this input

State			- ON/OFF state of the ramping switch

TimeUp			- Ramp up time

TimeDown		- Ramp down time

#### 8.5.2.2	Submix group master gain

| Description | Object: VMIX.SUB#<n> This property is read-and-write. It allows reading the current and setting the desired master gain of a submix group |
| --- | --- |
| Property name | Master_Gain |
| Data type | Decimal fixed point |
| Value range | - Represents the gain value in dB - The protocol allows arbitrary values, limited only by the data type - The surface application will trim the values received with SET commands to the following meaningful range: –80 .. +10dB, OFF - All values outside this numeric range will be interpreted by the application either as the maximal gain +10, or OFF, respectively - The gain resolution in surface application is 0.1dB. All higher precision values will be rounded to the closest value of the 0.1dB grid |

#### 8.5.2.3	Input source

RQ 4519

| Description | Object: VMIX.SUB#<n>.IN#<k> This property is read-and-write. It can be used to specify the input source to be connected, or to read the actually connected source. |
| --- | --- |
| Property name | input_uri |
| Data type | Literal |
| Value range | 1) Generic URI syntax. 2) A shortened form of the generic URI is allowed, skipping the URI scheme name and starting with the IP address. Since the scheme name cannot begin with a number, the shortened form can be unambiguously recognized. 24 Feb 2009: Supporting the shortened form is presently the only mandatory requirement. |

#### 8.5.2.4	Input gain

| Description | Object: VMIX.SUB#<n>.IN#<k> This property is read-and-write. It allows reading the current and setting the desired gain on an individual input to a submix group |
| --- | --- |
| Property name | Gain |
| Data type | Decimal fixed point |
| Value range | - Represents the gain value in dB - The protocol allows arbitrary values, limited only by the data type - The surface application will trim the values received with SET commands to the following meaningful range: –80 .. +10dB, OFF - All values outside this numeric range will be interpreted by the application either as the maximal gain +10, or OFF, respectively - The gain resolution in surface application is 0.1dB. All higher precision values will be rounded to the closest value of the 0.1dB grid |

#### 8.5.2.5	Input ramp switch state

| Description | Object: VMIX.SUB#<n>.IN#<k> This property is read-and-write. It allows reading the current and setting the desired state of the ramp switch on an individual input to a submix group. Changing this switch state does not change the gain to the opposite state instantly. It only triggers a timed transition between the ON and OFF states |
| --- | --- |
| Property name | State |
| Data type | Named constant |
| Value range | Enumerated: ON,OFF |

#### 8.5.2.6	Input ramp up/down time

| Description | Object: VMIX.SUB#<n>.IN#<k> These properties are read-and-write. They allows reading the current and setting the desired duration of the timed transitions between the ON and OFF states of the ramp switch. |
| --- | --- |
| Property name | TimeUp TimeDown |
| Data type | Decimal fixed point |
| Value range | - Represents the time value in seconds - The protocol allows arbitrary values, limited only by the data type - The surface application will trim the values received with SET commands to the following meaningful range: 0 .. 60 - All values outside this numeric range will be interpreted by the application either as the minimal time 0 sec, or maximal time 60 sec, respectively - The time resolution in surface application is 0.1 sec. All higher precision values will be rounded to the closest value of the 0.1 sec grid |

### 8.5.3	Commands and events

#### 8.5.3.1	Legend

C	Client		Represents the external entity communicating with the resource

S	Server		Represents the resource as a subject of CRCP

R	Resource	Represents the internal algorithms of the resource

#### 8.5.3.2	Subscribe to V-Mixer control parameter changes

To receive asynchronous notifications about V-Mixer parameter value changes, the remote application must subscribe to the VMIX object.

C: SUB VMIX $ACK $TRXI=555

S: ACK $TRXI=555

#### 8.5.3.3	Input source connection

RQ 4519

Use the SET command to connect or disconnect the input source. The shortened notation is used, where the URI scheme name is skipped. Since this is supposed to be operated over TCP unicast, reliable delivery is assumed, and no acknowledgement is requested:

C: SET VMIX.SUB#1.IN#5 input_uri=”239.192.0.115:4005”

S:

Use the GET command to learn the actually connected source.

C: GET VMIX.SUB#1.IN#5 input_uri

S: INDI VMIX.SUB#1.IN#5 input_uri=”239.192.0.115:4005”

Empty URI value would disconnect the input:

C: SET VMIX.SUB#1.IN#5 input_uri=””

S:

#### 8.5.3.4	Set V-Mixer control parameter by remote application

The remote application will use SET commands to control V-Mixer parameter values in the console or engine. Since this is supposed to be operated over TCP unicast, reliable delivery is assumed, and no acknowledgement is requested.

Set a group master gain, success:

C: SET VMIX.SUB#1 Master_Gain=<new gain value>

S:

Set an input ramp switch state, success:

C: SET VMIX.SUB#1.IN#5 State=ON

S:

Set an input ramp switch state, failure, transaction ID was not included:

C: SET VMIX.SUB#1.IN#5 State=ON

S: ACK $STATUS=ERROR

Set an input ramp switch state, failure, transaction ID was included:

C: SET VMIX.SUB#1.IN#5 State=ON $TRXI=555

S: ACK $TRXI=555 $STATUS=ERROR

Note:	General decision to use transaction ID and error codes, as well as their detail level and concrete values is up to implementation convention.

#### 8.5.3.5	Read V-Mixer control parameter by remote application

The remote application can read V-Mixer parameter values at any time using GET commands. Subscription is not required to receive value indications in response to GET commands.

Read an input ramp switch state:

C: GET VMIX.SUB#1.IN#5 State

S: INDI VMIX.SUB#1.IN#5 State=ON

#### 8.5.3.6	Notify remote application about V-Mixer control parameter change

The resource may change V-Mixer parameters locally. In such cases, the resource will notify the remote application about the changes by means of EVENT messages. To receive notifications, it is required and sufficient that the client were subscribed to the properties of the V-Mixer object. Only the actually changed properties will be included in the notification.

User: request changing a V-Mixer input state via the resource local interface

R: change the input state

S: EVENT VMIX.SUB#1.IN#5 State=<new state>

C: (update the display)

## 8.6	V-Mode

### 8.6.1	Objects

The V-Mode resource is represented by a number of identical V-Mode objects. Each of them is a non-structured,  single-level item.

V-Mode object

The class name for V-Mode object is:		VMODE

The entity identifier is an index number, and a full Object ID for a V-Mode object is:

VMODE#<number>

Example:					VMODE#3

### 8.6.2	Properties

#### 8.6.2.1	Overview

This set of properties covers the presently implemented source selection and signal profiling capabilities of the V-Mode resource, excepting the external input stream connections (address, name), which are controlled via LWRP.

V-Mode properties

input_uri		- The uniform resource identifier of the input source	RQ 4519

In_Select		- State of the input source selector

Audio_Mode		- Audio transfer mode

#### 8.6.2.2	Input source

RQ 4519

| Description | Object: VMODE#<n> This property is read-and-write. It can be used to specify the input source to be connected, or to read the actually connected source. |
| --- | --- |
| Property name | input_uri |
| Data type | Literal |
| Value range | 1) Generic URI syntax. 2) A shortened form of the generic URI is allowed, skipping the URI scheme name and starting with the IP address. Since the scheme name cannot begin with a number, the shortened form can be unambiguously recognized. 24 Feb 2009: Supporting the shortened form is presently the only mandatory requirement. |

#### 8.6.2.3	Input source selector state

| Description | Object: VMODE#<n> This property is read-and-write. It allows reading the current and setting the desired state of the input source selector switch. Changing this switch state does not connect/disconnect the external input audio stream. It only points the V-Mode input to a certain internal signal tap. |
| --- | --- |
| Property name | In_Select |
| Data type | Named constant |
| Value range | RQ 3419 Enumerated: PGM1, PGM1PROC, PGM2, PGM3, PGM4, RECORD, PHONE, AUXA, AUXB, AUXC, AUXD, VMIX_MAIN, VMIX_SUB01, VMIX_SUB02, VMIX_SUB03, VMIX_SUB04, VMIX_SUB05, VMIX_SUB06, VMIX_SUB07, VMIX_SUB08, EXT |

#### 8.6.2.4	Audio transfer mode

| Description | Object: VMODE#<n> This property is read-and-write. It allows reading the current and setting the desired audio transfer mode. |
| --- | --- |
| Property name | Audio_Mode |
| Data type | Named constant |
| Value range | Enumerated: PASS_LR, PASS_8CH, UPMIX_L, UPMIX_R, UPMIX_sumLR3, UPMIX_LRto51, DOWNMIX_51toLR, SPLIT_sumLR3toL, SPLIT_sumLR3toR, SPLIT_sumLR6toL, SPLIT_sumLR6toR, COMB_LL, COMB_LR, COMB_RL, COMB_RR, COMB_LR51, COMB_51LR |

### 8.6.3	Commands and events

#### 8.6.3.1	Legend

C	Client		Represents the external entity communicating with the resource

S	Server		Represents the resource as a subject of CRCP

R	Resource	Represents the internal algorithms of the resource

#### 8.6.3.2	Subscribe to V-Mode control parameter changes

To receive asynchronous notifications about V-Mode parameter value changes, the remote application must subscribe to the VMODE objects.

The most basic implementation is only required to support a common subscription to all VMODE entities together. In this case the subscription request would not specify any concrete entity number:

C: SUB VMODE $ACK $TRXI=555

S: ACK $TRXI=555

#### 8.6.3.3	Input source connection

RQ 4519

Use the SET command to connect or disconnect the input source. The shortened notation is used, where the URI scheme name is skipped. Since this is supposed to be operated over TCP unicast, reliable delivery is assumed, and no acknowledgement is requested:

C: SET VMODE#1 input_uri=”239.192.0.115:4005”

S:

Use the GET command to learn the actually connected source.

C: GET VMODE#1 input_uri

S: INDI VMODE#1 input_uri=”239.192.0.115:4005”

Empty URI value would disconnect the input:

C: SET VMODE#1 input_uri=””

S:

#### 8.6.3.4	Set V-Mode control parameter by remote application

The remote application will use SET commands to control V-Mode parameter values. Since this is supposed to be operated over TCP unicast, reliable delivery is assumed, and no acknowledgement is requested.

Set input source selector, success:

C: SET VMODE#1 In_Select=PGM1

S:

Set audio transfer mode, success:

C: SET VMODE#1 Audio_Mode=PASS_LR

S:

Failed attempt to set undefined audio transfer mode, transaction ID was not included:

C: SET VMODE#1 Audio_Mode=PASS_ABC

S: ACK $STATUS=ERROR

Failed attempt to set undefined audio transfer mode, transaction ID was included:

C: SET VMODE#1 Audio_Mode=PASS_ABC $TRXI=555

S: ACK $TRXI=555 $STATUS=ERROR

Note:	General decision to use transaction ID and error codes, as well as their detail level and concrete values is up to implementation convention.

#### 8.6.3.5	Read V-Mode control parameter by remote application

The remote application can read V-Mode parameter values at any time using GET commands. Subscription is not required to receive value indications in response to GET commands.

Read input source selector state:

C: GET VMODE#1 In_Select

S: INDI VMODE#1 In_Select=PGM1

#### 8.6.3.6	Notify remote application about V-Mode control parameter change

The resource may change V-Mode parameters locally. In such cases, the resource will notify the remote application about the changes by means of EVENT messages. To receive notifications, it is required and sufficient that the client were subscribed to the properties of the V-Mode object. Only the actually changed properties will be included in the notification.

User: request changing a V-Mode input selector state via the resource local interface

R: change the input selector state

S: EVENT VMODE#1 In_Select=PGM1

C: (update the display)

## 8.7	Special provision for operations over UDP multicast

The channel-oriented operations substantially rely on the broadcast (multicast) type of distribution, which does not require the sender to be aware of presence and identity of concrete target objects.

Neither TCP point-to-point, nor UDP multicast transport immediately ensure reliable point-to-multipoint delivery. According to the original concept, a special interim connection management layer would have to deal with that, but it does not exist at the moment of designing this part of the protocol (07 Apr 2006). Therefore the minimally required mechanisms are being implemented directly at the application protocol layer.

The approach taken here is based on the assumption that the operations will be carried out in a congestion and error free LAN environment:

- UDP multicast will be used as the delivery method

- To ensure reliable delivery, every UDP message will be sent twice. In absence of hardware defects, the probability to lose two messages at a time on a LAN is small sufficiently, to be ignored.

- Mandatory message identifiers will be included to recognize the duplicates – the same ID will be used for the both copies of a message. If the original message was successfully received, its copy will be simply ignored.

# 9.	Resource Discovery Protocol

## 9.1	Application

The Resource Discovery Protocol (RDP) is intended to provide external entities with data about the resources available in the unit. It has read-only access to the stored configuration data.

RDP supports basic access control using a simple login procedure.

Note:	Presently this protocol is only specified to provide responses to a few specific data requests. It may be extended in future with more data items and also to include resource advertising logic.

## 9.2	Resource data objects

- Presently a single entity in a single object class is defined – configuration data. At the root level it represents taken together all the configuration data components existing in the unit.

- The class name for the configuration data object is:

ConfigData

- As long as there is just one such entity per unit, no entity identifier is needed.

- If needed, individual components of the configuration data object would be addressable as nested objects, lower by one structure level. For example:

ConfigData		- Represents all configuration data components

ConfigData.Source#227	- An individual source profile referenced by ID

ConfigData.Show#5	- An individual show profile referenced by ID

## 9.3	Properties

### 9.3.1	ShowProfList: Show profile list

| Description | The ShowProfList property is supported by console application. It is attached at the top level of ConfigData object, and is read-only. Reading it returns the list of all show profiles from the stored data table. Each list entry consists of a show profile ID and name. |
| --- | --- |
| Property name | ShowProfList |
| Data type | Encapsulated data |
| Value range | XML document with the following structure: <List> <ShowProfile> <ID>(profile ID)</ID> <Name>(profile name)</Name> </ShowProfile> ... </List> Empty list: <List></List> |

### 9.3.2	src_list: Source profile list

RQ 3692

| Description | This is the complete unfiltered list of all source profiles stored in the console configuration data. It is attached at the top level of ConfigData object, and is read-only. Each list entry consists of a source profile ID, source profile name, and the Livewire channel number of the associated physical source.. |
| --- | --- |
| Property name | src_list |
| Data type | Encapsulated data |
| Value range | XML document with the following structure: <list> <src> <id>(profile id)</id> <name>(profile name)</name> <lwch>(livewire channel number)</lwch> </src> ... <lList> Empty list: <list></list> |

### 9.3.3	GpioList: GPIO resource list

| Description | The GpioList property is supported by GPIO application. It is attached at the top level of ConfigData object, and is read-only. Reading it returns the list of all GPIO circuits from the stored data table. Each list entry consists of a circuit ID and function description. |
| --- | --- |
| Property name | GpioList |
| Data type | Encapsulated data |
| Value range | Document with the following structure: <List> <Input> <Channel>(LW channel number) </Channel> <Circuit>(Circuit number)</Circuit> <Function>(Textual description)</Function> </Input> ... <Output> <Channel>(LW channel number)</Channel> <Circuit>(Circuit number)</Circuit> <Function>(Textual description)</Function> </Output> ... </List> Empty list: <List></List> |

### 9.3.4	console_hardware: Console hardware description

RQ 4610

| Description | The console_hardware property is attached at the top level of ConfigData object, and is read-only. It displays a detailed description of the actually installed console hardware in a structured form, arranged on the module/resource basis. |
| --- | --- |
| Property name | console_hardware |
| Data type | Encapsulated data |
| Value range | Note, 18 Sep 2009: Presently, due to implementation limitations, only the fader channel numbers are included, and they are shown all belonging to one module with unknown module ID. The data content is to be extended in future, to provide all the needed details about the console hardware. The console hardware description is a document with the following structure: <slot> <id>-1</id> <resource> <type>fader</type> <id>(Fader number)</id> </resource> ... </slot> ... <slot> ... </slot> - There will be a <slot> group per each installed module - The <id> value matches the actual hardware ID of the module. The value –1 denotes an unknown module ID. - There will be a <resource> group per each resource existing on the module. Examples of resources: fader, LCD legend push button, phone system controller panel. |

## 9.4	Resource discovery commands

### 9.4.1	Obtain a list of show profiles from console configuration data

C: GET ConfigData ShowProfList

S: INDI ConfigData ShowProfList=%BeginEncap%(list data goes here)%EndEncap%

Returning empty list:

S: INDI ConfigData ShowProfList=%BeginEncap%<List></List>%EndEncap%

### 9.4.2	Obtain a list of source profiles from console configuration data

RQ 3692

C: GET ConfigData src_list

S: INDI ConfigData src_list=%BeginEncap%

<list>
    <src>
        <id>1</id>
        <name>Goog_115</name>
        <lwch>115</lwch>
    </src>
    <src>
        <id>2</id>
        <name>Goog_116</name>
        <lwch>116</lwch>
    </src>
</list>
%EndEncap%

Returning empty list:

S: INDI ConfigData src_list=%BeginEncap%<List></List>%EndEncap%

### 9.4.3	Obtain a list of GPIO resources from GPIO unit configuration data

C: GET ConfigData GpioList

S: INDI ConfigData GpioList=%BeginEncap%(list data goes here)%EndEncap%

Returning empty list:

S: INDI ConfigData GpioList=%BeginEncap%<List></List>%EndEncap%

### 9.4.4	Obtain console hardware description from configuration data

RQ 4610

C: GET ConfigData console_hardware

S: INDI ConfigData console_hardware=%BeginEncap%

<slot>
    <id>-1</id>
    <resource>
        <type>fader</type>
        <id>1</id>
    </resource>
    ...
    <resource>
        <type>fader</type>
        <id>14</id>
    </resource>
</slot>
%EndEncap%

Returning empty description:

S: INDI ConfigData console_hardware=%BeginEncap%<slot></slot>%EndEncap%

## 9.5	Message sequence examples

Client logs in to the console config data storage. Only basic login is presently defined, no access levels.

C: LOGIN ConfigData USER=MIKE,PWD=AXIA $ACK $TRXI=555

S: ACK $TRXI=555 $STATUS=OK

Client obtains the show profile list …

C: GET ConfigData ShowProfList

S: INDI ConfigData ShowProfList=%BeginEncap%

<List>

<ShowProfile>

<ID>1</ID>

<Name>Default</Name>

</ShowProfile>

<ShowProfile>

<ID>2</ID>

<Name>Mike’s Morning</Name>

</ShowProfile>

<ShowProfile>

<ID>3</ID>

<Name>Night Music</Name>

</ShowProfile>

</List>

%EndEncap%

… and triggers changing the active profile on console using the console remote control protocol

C: SET AppControl ShowProfID=3

Console: start loading the new profile

S: EVENT AppControl ShowProfID=3,ShowProfName=”Night Music”,ShowProfStat=LOADING

Console: loading the new profile completed

S: EVENT AppControl ShowProfID=3,ShowProfName=”Night Music”,ShowProfStat=READY

Client obtains the GPIO resource list from a GPIO unit …

C: GET ConfigData GpioList

S: INDI ConfigData GpioList=%BeginEncap%

<List>

<Input>

<Channel>1002</Channel>

<Circuit>1</Circuit>

<Function>ST monitor: MUTE command</Function>

</Input>

<Input>

<Channel>1002</Channel>

<Circuit>2</Circuit>

<Function>ST monitor: DIM command</Function>

</Input>

<Input>

<Channel>1002</Channel>

<Circuit>3</Circuit>

<Function>ST monitor: Timer trigger command</Function>

</Input>

<Output>

<Channel>1002</Channel>

<Circuit>1</Circuit>

<Function>ST monitor: ON AIR lamp</Function>

</Output>

<Output>

<Channel>1002</Channel>

<Circuit>2</Circuit>

<Function>ST monitor: DIM lamp</Function>

</Output>

<Output>

<Channel>1002</Channel>

<Circuit>3</Circuit>

<Function>ST monitor: Timer trigger output</Function>

</Output>

<Output>

<Channel>1002</Channel>

<Circuit>5</Circuit>

<Function>ST monitor: TALK ACTIVE lamp</Function>

</Output>

</List>

%EndEncap%

… and turns on the TALK ACTIVE lamp

C: SET CH#1002.GPO#5 PULSE=HIGH

# 10.	Unit Supervision

RQ 3994

## 10.1	Application

This protocol module is intended to be used for general control and monitoring. It deals with the following information:

- Operation states

- Device parameters

- Application type, software version, and capability set

In order to access the unit supervision services, a unicast TCP connection should be opened to port 4010 (decimal).

## 10.2	Objects

### 10.2.1	Supervision entity

At the root level the supervision entity represents all components belonging to the unit supervision context. No entity identifier is needed at this level.

The class name for the supervision entity is:

supv

Subsystems or modules, if such exist, may be represented by nested objects, at the next lower structure level. If more than one instance of a class is possible, an entity identifier should be used. Modules of an Element console could serve as a general example (not for exact and immediate implementation yet):

supv.mod#1

## 10.3 Properties

### 10.3.1	Overview

The present set of properties covers the immediately requested unit supervision capabilities, but it is not closed. More properties may be added at any time to include support for other functions.

The following universal properties are defined, to be supported by all Livewire devices:

device_type		- Principal function of the unit (is it a console, a mix engine, a node, etc)

version		- Software system version ID

state			- Supervision state

In addition to the universal properties, each device type may have other ones, associated with its specific function. Presently the following device-specific properties are defined:

Mix engine:

console_ip		- IP of the current controlling console

console_type		- Console type (SmartSurface, Element, etc)

audio_mix_mode	- Audio mixing mode (Stereo or 8-channel)

### 10.3.2	Device type

| Description | Objects: supv Devices supporting this property: all types This property reports the principal function of the unit. It is read-only. Equivalent data items on other interfaces: - LWRP on port 93: DEVN parameter in the VER message |
| --- | --- |
| Property name | device_type |
| Data type | Named constant |
| Value range | Enumerated: ELEMENT, ENGINE, OMNIA8X, ZIPORT, LIVEIO, GPIO Notes: ZIPORT – Zephyr iPort LIVEIO – used for all types of Axia audio nodes |

### 10.3.3	Software version ID

| Description | Objects: supv Devices supporting this property: all types This property reports the software system version ID. It is read-only. Equivalent data items on other interfaces: - LWRP, port 93: SYSV parameter in the VER message - CMsg console-engine supervision, port 4002: SUPV.VERS |
| --- | --- |
| Property name | version |
| Data type | Literal constant |
| Value range | In order to ensure uniform machine readable formatting throughout all the Axia system, the version ID must be in one of the following formats, as defined in “Axia Management Console (AMC) Support Definition Document version 1.0 revision 12” <number1>.<number2>.<number3>.<number4>….<numberN> <number1>.<number2>.<number3>.<number4>….<numberN><letter> Examples: version=“2.4.8.12” version=“1.0.2r” |

### 10.3.4	Supervision state

| Description | Objects: supv Devices supporting this property: all types This property reports the actual operation state. It is read-only. Note: A state transition in the general case may be a complex operation, which can last in time and may fail in the end. Therefore using this property to initiate state changes may lead to difficult handling and unclean design. For this purpose a dedicated target state command property may be introduced, when needed. Equivalent data items on other interfaces: - CMsg console-engine supervision, port 4002: SUPV.STAT |
| --- | --- |
| Property name | state |
| Data type | Named constant |
| Value range | Enumerated: STARTUP, READY, WORKING, SHUTDOWN, ERROR |

### 10.3.5	Controlling console IP address

| Description | Objects: supv Devices supporting this property: mix engine This property reports the IP address of the console currently controlling this mix engine. It is read-only. Equivalent data items on other interfaces: - CMsg console-engine supervision, port 4002: SUPV.SSIP |
| --- | --- |
| Property name | console_ip |
| Data type | Literal constant |
| Value range | Text formatted using the common IP address notation – four 8-bit numbers in decimal form, separated with dots: console_ip=”192.168.12.148” |

### 10.3.6	Console type

| Description | Objects: supv Devices supporting this property: mix engine This property reports the console type currently controlling this mix engine. It is read-only. Equivalent data items on other interfaces: - CMsg console-engine supervision, port 4002: SUPV.APPT |
| --- | --- |
| Property name | console_type |
| Data type | Named constant |
| Value range | Enumerated: SMARTSURFACE, ELEMENT |

### 10.3.7	Audio mixing mode

| Description | Objects: supv Devices supporting this property: mix engine This property reports the audio mixing mode selected by the console currently controlling this mix engine. It is read-only. Equivalent data items on other interfaces: - CMsg console-engine supervision, port 4002: SUPV.AMXM |
| --- | --- |
| Property name | audio_mix_mode |
| Data type | Named constant |
| Value range | Enumerated: STEREO, SURR51LR |

## 10.4	Message sequences

### 10.4.1	Overview

04 Jul 2008:

At the moment of writing this, the required minimum is to allow external clients to obtain the unit status information. This will be done by querying the “state” property. For all device types, it is mandatory to respond with the three universal properties – device type, version, and state.

It is expected that the great majority of all status polling transactions will only use the state value. Therefore, to optimize the operation, the “state” property must always go first in the response message.

In addition to this, devices are allowed, but not universally required, to include in the status responses also their type-specific properties. The type-specific properties, if included, must always follow after the mandatory universal properties. The concrete property set to be included in each particular case is left to be determined by an  implementation convention.

Besides querying the unit status, the protocol design allows individually reading each of the other properties. Supporting this is not a mandatory requirement at the moment.

Another possibly useful future extension could be querying consolidated unit data. Since different devices are going to have rather different data sets, using the form of an XML document seems reasonable for this purpose.

### 10.4.2	Unit status polling – the minimal response

04 Jul 2008:   This is the only mandatory requirement now.

In response to querying its status, the device is reporting only the three universal properties:

C: GET supv state

S: INDI supv state=WORKING, device_type=GPIO, version=”2.5.2g”

### 10.4.3	Unit status polling – extended response

04 Jul 2008:   This may be implemented according to a local convention, but is not universally required

In response to querying its status, the mix engine is including its type-specific data:

C: GET supv state

S: INDI supv state=WORKING, device_type=ENGINE, version=”2.4.9a”,

console_ip=”192.168.12.148”, console_type=ELEMENT, audio_mix_mode=STEREO

### 10.4.4	Reading an individual property

04 Jul 2008:   This may be implemented according to a local convention, but is not universally required

C: GET supv version

S: INDI supv version=”2.5.2g”

### 10.4.5	Reading bulk data

04 Jul 2008:   Not to be implemented now.

This is an illustrative example of a possible future extension mechanism, not a final specification yet.

C: GET supv unit_data

S: INDI supv unit_data=%BeginEncap%

<Data>

<state>WORKING</state>

<device_type>ENGINE</device_type>

<version>2.4.9a</version>

<console_ip>192.168.12.148</console_ip>

<console_type>ELEMENT</console_type>

<audio_mix_mode>STEREO</audio_mix_mode>

</Data>

%EndEncap%

- Appendix 1.	Global status codes

| Numeric value | Name | Description |
| --- | --- | --- |
| 0 | OK | OK |
| Range 1..99: Objects and properties – ID and value related |  |  |
| 1 | UNKNOWN_PROPERTY | Unknown property |
| 2 | INVALID_VALUE | Invalid value |
| 7 | UNKNOWN_OBJECT | Unknown object |
| Range 100..199: Session management related |  |  |
| 100 | AUTHORIZATION_FAILURE | Authorization failure |

- Appendix 2.	Source status codes used in Element

Note, 18 Sep 2009:	The list below was extracted from Element documentation on the indicated date. It is included in this document for quick reference, although it may temporarily go out of sync with the primary document, as the Element application develops further.

| Status code | Description | Notes |
| --- | --- | --- |
| 0 | Status is not assigned, it is unknown |  |
| 1 | Channel is idle because no source was specified |  |
| 2 | Source is successfully loaded | Used only with monitor and aux return inputs. Not used with fader channels. |
| 3 | Can’t clear the channel because it is ON | Used only with fader channels. Not used with monitor and aux return inputs. |
| 4 | Can’t load the source because the channel is ON | Used only with fader channels. Not used with monitor and aux return inputs. |
| 5 |  | Not used anymore |
| 6 |  | Not used anymore |
| 7 | Can’t load the source because it is currently not active (no advertisement) |  |
| 8 | Can’t load the source because the specified source profile ID does not exist |  |
| 9 |  | Not used anymore |
| 10 | Can’t load the source because the channel is ON, and the source is currently not active (no advertisement) | Used only with fader channels. Not used with monitor and aux return inputs. |
| 11 |  | This is an interim state |
| 12 |  | This is an interim state |
| 21 | Source is loaded on fader in listen-only mode | Used only with fader channels. Not used with monitor and aux return inputs. |
| 22 | Source is acquired for full control and loaded on fader | Used only with fader channels. Not used with monitor and aux return inputs. |
| 23 | Source is acquired for full control and loaded on fader, but failed to create the backfeed | Used only with fader channels. Not used with monitor and aux return inputs. |
| 24 | Source is locked for full control elsewhere; it is loaded on fader, but forced to listen-only mode | Used only with fader channels. Not used with monitor and aux return inputs. |
| 25 | Source could not be locked for full control because of an error; it is loaded on fader, but forced to listen-only mode | Used only with fader channels. Not used with monitor and aux return inputs. |
| 26 | Source got unlocked after it was already loaded on fader; it stays forced to listen-only mode | Used only with fader channels. Not used with monitor and aux return inputs. |
| 27 | Source allocation request is pending – the source either returned “free” status, or the response timed out | Used only with fader channels. Not used with monitor and aux return inputs. |
| 128 | General source error. This is the common status for all kinds of abnormal situations related to source data retrieval, allocation status, owner ID mismatch, application procedure or communication errors |  |
| 129 | Unrecognized source loading error. This is the common status for all kinds of unrecognized abnormal situations encountered during the attempt to load source. |  |
| 130 | Can’t clear the channel, reason unknown. Engine failed to execute some of the commands that would clear the channel settings, and no detailed error analysis is available. | Used only with fader channels. Not used with monitor and aux return inputs. |